CCNA + CCNP Complete Study Notes

Beginner to Advanced • Theory & Practical • All Exam Topics

CCNA 200-301  |  CCNP ENCOR 350-401

29 Comprehensive Chapters  •  Full Cisco IOS Lab Commands  •  Reference Tables

TABLE OF CONTENTS

| # | Chapter | Topic | Part |
| --- | --- | --- | --- |
| 1 | Introduction to Networking | Network types, topologies, devices, cables | Part 1: Fundamentals |
| 2 | OSI & TCP/IP Models | All 7 layers, encapsulation, TCP vs UDP | Part 1: Fundamentals |
| 3 | IPv4 Addressing | Address classes, private IPs, VLSM subnetting | Part 1: Fundamentals |
| 4 | IPv6 Addressing | Format, types, EUI-64, SLAAC, DHCPv6 | Part 1: Fundamentals |
| 5 | Ethernet & Switch Operation | Frames, MAC addresses, forwarding methods | Part 2: Switching |
| 6 | VLANs & Trunking | 802.1Q, access/trunk ports, inter-VLAN routing | Part 2: Switching |
| 7 | Spanning Tree Protocol | STP, RSTP, MSTP, PortFast, BPDU Guard | Part 2: Switching |
| 8 | EtherChannel | LACP, PAgP, port-channel configuration | Part 2: Switching |
| 9 | Layer 2 Security | Port Security, DHCP Snooping, DAI, VLAN hopping | Part 2: Switching |
| 10 | Routing Fundamentals | Routing table, AD, static routes, longest match | Part 3: Routing |
| 11 | OSPF | LSA, areas, adjacency, DR/BDR, multi-area, auth | Part 3: Routing |
| 12 | EIGRP | DUAL, FD/RD, successor/FS, named mode | Part 3: Routing |
| 13 | BGP | eBGP/iBGP, path attributes, best-path selection | Part 3: Routing |
| 14 | Redistribution & PBR | Route redistribution, policy-based routing | Part 3: Routing |
| 15 | FHRP — HSRP/VRRP/GLBP | First Hop Redundancy, virtual IPs, preempt | Part 3: Routing |
| 16 | Access Control Lists | Standard/Extended, named, IPv6 ACLs | Part 4: Services |
| 17 | NAT & PAT | Static, Dynamic, Overload, verification | Part 4: Services |
| 18 | DHCP, DNS & NTP | DORA, DHCP relay, DNS records, NTP stratum | Part 4: Services |
| 19 | Network Security & AAA | IOS hardening, SSH, TACACS+, RADIUS | Part 4: Services |
| 20 | Quality of Service | DSCP, CoS, LLQ, CBWFQ, traffic shaping | Part 4: Services |
| 21 | WAN Technologies | MPLS, SD-WAN, DSL, fiber, cellular, satellite | Part 5: WAN |
| 22 | VPN Technologies | IPsec, IKE, GRE, DMVPN, FlexVPN | Part 5: WAN |
| 23 | Wireless Networking | 802.11 standards, WPA3, WLC, CAPWAP | Part 5: WAN |
| 24 | SNMP, Syslog & NetFlow | SNMPv3, severity levels, IP SLA, tracking | Part 6: CCNP Adv. |
| 25 | Multicast Routing | IGMP, PIM, RP, multicast addressing | Part 6: CCNP Adv. |
| 26 | SD-WAN & VRF | Viptela architecture, VRF-Lite, VRF routing | Part 6: CCNP Adv. |
| 27 | Network Automation | NETCONF, RESTCONF, Ansible, Python, YANG | Part 6: CCNP Adv. |
| 28 | Troubleshooting | Methodologies, commands, common issues | Part 7: Reference |
| 29 | Quick Reference | Ports, codes, subnets, attacks, study roadmap | Part 7: Reference |

PART 1: NETWORK FUNDAMENTALS

# Chapter 1: Introduction to Networking

Networking is the practice of connecting computing devices so they can communicate and share resources. Understanding networking begins with understanding how data moves from one device to another — through cables, wireless signals, and software protocols.

## 1.1 Types of Networks

| Type | Full Name | Scope | Example |
| --- | --- | --- | --- |
| PAN | Personal Area Network | Within 10 meters | Bluetooth headset to phone |
| LAN | Local Area Network | Office / home building | Devices on same floor |
| MAN | Metropolitan Area Network | City or campus | University campus network |
| WAN | Wide Area Network | Across cities / countries | The Internet, leased lines |
| CAN | Campus Area Network | Multiple buildings, one campus | Corporate HQ campus |
| SAN | Storage Area Network | Dedicated storage network | Data center disk arrays |
| WLAN | Wireless LAN | Same as LAN, wireless medium | Wi-Fi home network |

## 1.2 Network Topologies

| Topology | Description | Pros | Cons |
| --- | --- | --- | --- |
| Bus | All devices on a single shared cable | Cheap, simple | Single point of failure; collisions |
| Ring | Devices connected in a loop | No collisions with token passing | Slow; one break = full outage |
| Star | All devices connect to central switch/hub | Easy to manage & troubleshoot | Switch is single point of failure |
| Mesh (Full) | Every device connected to every other | Highest redundancy | Very expensive; complex |
| Mesh (Partial) | Some redundant links, not all | Balance of cost & redundancy | Less fault-tolerant than full mesh |
| Hybrid | Combination of multiple topologies | Flexible & scalable | Complex to design |
| Point-to-Point | Direct link between two devices | Dedicated bandwidth | Not scalable |

## 1.3 Network Devices

| Device | OSI Layer | Function | Key Characteristic |
| --- | --- | --- | --- |
| Hub | Layer 1 (Physical) | Repeats signal to all ports | One collision domain; one broadcast domain |
| Switch | Layer 2 (Data Link) | Forwards frames by MAC address | One collision domain per port; one broadcast domain |
| Bridge | Layer 2 (Data Link) | Segments network into collision domains | Software-based, slower than switch |
| Router | Layer 3 (Network) | Routes packets between networks by IP | Separates broadcast domains; creates new subnet per interface |
| Layer 3 Switch | Layer 2 & 3 | Switch + routing capability | High-speed inter-VLAN routing |
| Firewall | Layer 3–7 | Filters traffic based on rules | Stateful inspection; NAT; VPN |
| AP (Access Point) | Layer 1–2 | Provides wireless access to LAN | Connects wireless clients to wired network |
| Repeater | Layer 1 (Physical) | Regenerates and amplifies signal | Extends cable distance only |
| Modem | Layer 1–2 | Modulates/demodulates signal for WAN | Converts digital to analog and back |
| NIC | Layer 1–2 | Network Interface Card — connects device to network | Has a unique 48-bit MAC address |

## 1.4 Network Cables & Media

| Cable Type | Max Speed | Max Length | Standard | Use Case |
| --- | --- | --- | --- | --- |
| Cat 5e (UTP) | 1 Gbps | 100 m | 802.3ab | Older office LANs |
| Cat 6 (UTP) | 10 Gbps | 55 m (10G) / 100m (1G) | 802.3an | Modern office LANs |
| Cat 6a (UTP) | 10 Gbps | 100 m | 802.3an | Enterprise floors |
| Cat 7 (STP) | 10 Gbps | 100 m | ISO 11801 | Data centers, shielded |
| Cat 8 (STP) | 40 Gbps | 30 m | 802.3bq | Short data center runs |
| Single-Mode Fiber | 100 Gbps+ | Up to 100 km | 802.3ae | Long-haul WAN, campus backbone |
| Multi-Mode Fiber | 10–100 Gbps | Up to 2 km | 802.3z | Short data center runs |
| Coaxial | Up to 1 Gbps | Varies | DOCSIS | Cable TV/internet (HFC) |

💡 NOTE Straight-through cable: PC→Switch, Switch→Router. Crossover cable: Switch→Switch, PC→PC. Modern devices use Auto-MDI/X — auto-detect, eliminating this distinction.

# Chapter 2: The OSI & TCP/IP Models

The OSI model is the universal reference framework for how network communication works. Every network concept can be mapped to one or more of its seven layers. The TCP/IP model is the practical implementation used on the internet today.

## 2.1 OSI Model — Detailed Layer-by-Layer

| Layer | Name | PDU | Function | Protocols / Devices |
| --- | --- | --- | --- | --- |
| 7 | Application | Data | Network services to end-user applications. User interface to the network. | HTTP, HTTPS, FTP, SFTP, SMTP, POP3, IMAP, DNS, DHCP, SNMP, Telnet, SSH, NTP |
| 6 | Presentation | Data | Data translation, encryption/decryption, compression. Converts formats for Application layer. | SSL/TLS, JPEG, PNG, MPEG, MP3, ASCII, Unicode, Base64 |
| 5 | Session | Data | Establishes, maintains, and terminates sessions between applications. Dialog control. | NetBIOS, RPC, SQL sessions, NFS, PAP, PPTP |
| 4 | Transport | Segment | End-to-end data delivery, segmentation, flow control, error recovery, multiplexing via ports. | TCP (reliable), UDP (best-effort) |
| 3 | Network | Packet | Logical addressing (IP), path determination, routing between networks. | IP (v4/v6), ICMP, OSPF, EIGRP, BGP, Router |
| 2 | Data Link | Frame | Physical addressing (MAC), framing, access to media, error detection (not correction). | Ethernet, Wi-Fi (802.11), PPP, HDLC, Frame Relay, Switch, Bridge, ARP |
| 1 | Physical | Bit | Transmission of raw bits over physical medium. Defines electrical, optical, mechanical specs. | Cables (UTP/fiber/coax), Hubs, Repeaters, NICs, Modems |

✅ TIP Memory Aid (Layer 7→1): 'All People Seem To Need Data Processing' ¦ (Layer 1→7): 'Please Do Not Throw Sausage Pizza Away'

## 2.2 TCP/IP Model vs OSI Model

| TCP/IP Layer | OSI Equivalent | Description |
| --- | --- | --- |
| Application | Layers 5, 6, 7 | Combines OSI Session, Presentation, and Application |
| Transport | Layer 4 | TCP and UDP — same as OSI Transport |
| Internet | Layer 3 | IP addressing and routing — same as OSI Network |
| Network Access | Layers 1, 2 | Combines OSI Physical and Data Link |

## 2.3 Encapsulation & De-encapsulation

Encapsulation is the process of adding headers (and sometimes trailers) as data travels DOWN the OSI stack from sender to receiver. De-encapsulation is the reverse — removing headers as data travels UP the stack on the receiving end.

| Layer | Action | Header Added | Result |
| --- | --- | --- | --- |
| 7–5 (Application) | Creates data | None | Raw application data |
| 4 (Transport) | Segmentation | TCP/UDP Header (src/dst port, seq#) | Segment |
| 3 (Network) | Adds IP info | IP Header (src/dst IP, TTL, protocol) | Packet |
| 2 (Data Link) | Framing | MAC Header + FCS Trailer | Frame |
| 1 (Physical) | Transmission | None (converts to bits) | Bits on the wire |

## 2.4 TCP vs UDP — Deep Dive

| Feature | TCP | UDP |
| --- | --- | --- |
| Full Name | Transmission Control Protocol | User Datagram Protocol |
| Connection Model | Connection-oriented (3-way handshake required) | Connectionless — no handshake |
| Reliability | Guaranteed delivery — uses ACKs | Best-effort — no ACKs |
| Ordering | Sequence numbers ensure in-order delivery | No ordering guarantee |
| Flow Control | Sliding window mechanism | None |
| Error Recovery | Retransmits lost segments | None — sender doesn't know |
| Congestion Control | Yes (slow start, congestion avoidance) | None |
| Speed | Slower due to overhead | Much faster — minimal overhead |
| Header Size | 20 bytes minimum (up to 60) | 8 bytes (fixed) |
| Use Cases | HTTP/S, FTP, SSH, Telnet, SMTP, IMAP | DNS, DHCP, VoIP, Video stream, SNMP, TFTP |

🔑 KEY CONCEPT TCP 3-Way Handshake: Client sends SYN → Server replies SYN-ACK → Client sends ACK. Connection is established. TCP 4-Way Teardown: FIN → ACK → FIN → ACK.

# Chapter 3: IPv4 Addressing

IPv4 uses 32-bit binary addresses written in dotted-decimal notation. Each address has two parts: Network (identifies the subnet) and Host (identifies the device within the subnet). The subnet mask determines where this boundary lies.

## 3.1 IPv4 Address Classes

| Class | First Octet Range | Default Mask | CIDR | # Networks | Hosts/Network | Purpose |
| --- | --- | --- | --- | --- | --- | --- |
| A | 1 – 126 | 255.0.0.0 | /8 | 126 | 16,777,214 | Very large enterprises |
| B | 128 – 191 | 255.255.0.0 | /16 | 16,384 | 65,534 | Medium-large organizations |
| C | 192 – 223 | 255.255.255.0 | /24 | 2,097,152 | 254 | Small networks |
| D | 224 – 239 | N/A | N/A | N/A | N/A | Multicast groups |
| E | 240 – 255 | N/A | N/A | N/A | N/A | Research / Experimental |

💡 NOTE 127.0.0.0/8 is reserved for loopback testing (127.0.0.1 = localhost). It is not part of Class A usable space.

## 3.2 Special & Private IPv4 Addresses

| Address / Range | Type | Description |
| --- | --- | --- |
| 0.0.0.0 | Unspecified | Means 'this host' or 'any address' |
| 127.0.0.0/8 | Loopback | Testing local IP stack — 127.0.0.1 most common |
| 10.0.0.0/8 | Private (Class A) | RFC 1918 — not routable on public internet |
| 172.16.0.0 – 172.31.255.255 /12 | Private (Class B) | RFC 1918 — not routable on public internet |
| 192.168.0.0/16 | Private (Class C) | RFC 1918 — most common for home/office LANs |
| 169.254.0.0/16 | APIPA | Auto-assigned when DHCP fails (Windows). Link-local. |
| 255.255.255.255 | Limited Broadcast | Broadcast to all hosts on local network |
| 224.0.0.0 – 239.255.255.255 | Multicast | One sender to multiple receivers |

## 3.3 Subnetting — Complete Guide

Subnetting divides a large network into smaller, more manageable sub-networks. This reduces broadcast traffic, improves security, and optimizes IP address usage.

| Prefix | Subnet Mask | Wildcard | # Hosts | # Subnets from /24 | Block Size |
| --- | --- | --- | --- | --- | --- |
| /24 | 255.255.255.0 | 0.0.0.255 | 254 | 1 | 256 |
| /25 | 255.255.255.128 | 0.0.0.127 | 126 | 2 | 128 |
| /26 | 255.255.255.192 | 0.0.0.63 | 62 | 4 | 64 |
| /27 | 255.255.255.224 | 0.0.0.31 | 30 | 8 | 32 |
| /28 | 255.255.255.240 | 0.0.0.15 | 14 | 16 | 16 |
| /29 | 255.255.255.248 | 0.0.0.7 | 6 | 32 | 8 |
| /30 | 255.255.255.252 | 0.0.0.3 | 2 | 64 | 4 — Point-to-Point Links |
| /31 | 255.255.255.254 | 0.0.0.1 | 2 (no Network/BC) | 128 | 2 — PPP RFC 3021 |
| /32 | 255.255.255.255 | 0.0.0.0 | 1 (host route) | 256 | 1 |

🔑 KEY CONCEPT Key Formulas: • Hosts per subnet = 2^(host bits) − 2 • Number of subnets = 2^(subnet bits) • Host bits = 32 − prefix length • Network Address: All host bits = 0 ¦ Broadcast: All host bits = 1

### 3.3.1 Subnetting Example — VLSM

Company has 172.16.0.0/16. Needs: Sales=100 hosts, Engineering=50 hosts, HR=25 hosts, Management=10 hosts, 3 WAN links (2 hosts each).

VLSM Strategy: Allocate largest subnets first, then smaller.

| Department | Hosts Needed | Subnet Chosen | Network Address | Broadcast | Usable Range |
| --- | --- | --- | --- | --- | --- |
| Sales | 100 | /25 (126 hosts) | 172.16.0.0/25 | 172.16.0.127 | 172.16.0.1 – .126 |
| Engineering | 50 | /26 (62 hosts) | 172.16.0.128/26 | 172.16.0.191 | 172.16.0.129 – .190 |
| HR | 25 | /27 (30 hosts) | 172.16.0.192/27 | 172.16.0.223 | 172.16.0.193 – .222 |
| Management | 10 | /28 (14 hosts) | 172.16.0.224/28 | 172.16.0.239 | 172.16.0.225 – .238 |
| WAN Link 1 | 2 | /30 (2 hosts) | 172.16.0.240/30 | 172.16.0.243 | 172.16.0.241 – .242 |
| WAN Link 2 | 2 | /30 (2 hosts) | 172.16.0.244/30 | 172.16.0.247 | 172.16.0.245 – .246 |
| WAN Link 3 | 2 | /30 (2 hosts) | 172.16.0.248/30 | 172.16.0.251 | 172.16.0.249 – .250 |

# Chapter 4: IPv6 Addressing

IPv6 was developed to solve IPv4 address exhaustion. It uses 128-bit addresses providing ~340 undecillion unique addresses. IPv6 also improves routing efficiency, eliminates NAT requirements, includes built-in IPsec, and introduces stateless address autoconfiguration (SLAAC).

## 4.1 IPv6 Address Format & Notation

| Rule | Example | Result |
| --- | --- | --- |
| Full 128-bit address | 2001:0DB8:0000:0000:0000:0000:0000:0001 | Uncompressed |
| Omit leading zeros in each group | 2001:DB8:0:0:0:0:0:1 | Shorter form |
| Replace one contiguous set of all-zero groups with :: | 2001:DB8::1 | Compressed (:: used once) |
| Loopback | 0000:0000:0000:0000:0000:0000:0000:0001 | Compressed: ::1 |
| Unspecified address | 0000:...0000 | Compressed: :: |

⚠️ WARNING The :: can only be used ONCE per address. Using it twice causes ambiguity about how many groups of zeros each :: represents.

## 4.2 IPv6 Address Types

| Address Type | Prefix | Scope | Description |
| --- | --- | --- | --- |
| Global Unicast (GUA) | 2000::/3 | Global (internet) | Publicly routable. Assigned by ISP. Equivalent to public IPv4. |
| Link-Local | FE80::/10 | Single link | Auto-configured on every IPv6 interface. Never routed beyond local segment. Required for neighbor discovery. |
| Unique Local (ULA) | FC00::/7 (FD00::/8 common) | Organization | Private use. Not routable on internet. Equivalent to RFC 1918 IPv4. |
| Multicast | FF00::/8 | Varies | One-to-many. Replaces IPv4 broadcast. Example: FF02::1 = all nodes. |
| Anycast | From unicast space | Global/local | Same address on multiple devices — packet delivered to nearest one. |
| Loopback | ::1/128 | Local host | Equivalent to 127.0.0.1. Used for testing local IPv6 stack. |
| Unspecified | ::/128 | Local | Indicates absence of address. Used in DHCP requests. |
| IPv4-Mapped | ::FFFF:0:0/96 | Local | Represents IPv4 address in IPv6 notation: ::FFFF:192.168.1.1 |

## 4.3 IPv6 Address Configuration Methods

| Method | Description | How it Works |
| --- | --- | --- |
| Static | Manually configured | Admin assigns address, prefix, gateway, DNS |
| SLAAC | Stateless Address Autoconfiguration | Device uses Router Advertisement (RA) to get prefix, then generates host part from MAC (EUI-64) or randomly (RFC 7217) |
| DHCPv6 Stateful | Full DHCPv6 | DHCPv6 server assigns full address + DNS — like IPv4 DHCP |
| DHCPv6 Stateless | SLAAC + DNS from DHCPv6 | Device does SLAAC for address, uses DHCPv6 only for DNS/options |

## 4.4 IPv6 EUI-64 — Interface ID Generation

EUI-64 generates the 64-bit interface ID from the 48-bit MAC address:

- Split MAC address in half — insert FF:FE in the middle

- Flip bit 7 (Universal/Local bit) of the first byte

| Step | Value |
| --- | --- |
| MAC Address | AA:BB:CC:DD:EE:FF |
| Split and insert FF:FE | AA:BB:CC:FF:FE:DD:EE:FF |
| Flip bit 7 of AA (0xAA = 10101010 → 10101000 = 0xA8) | A8:BB:CC:FF:FE:DD:EE:FF |
| Final Interface ID | A8BB:CCFF:FEDD:EEFF |
| Full IPv6 Link-Local | FE80::A8BB:CCFF:FEDD:EEFF/64 |

## 4.5 IPv6 Routing Configuration

! Enable IPv6 routing Router(config)# ipv6 unicast-routing ! Assign IPv6 address to interface Router(config)# interface GigabitEthernet0/0 Router(config-if)# ipv6 address 2001:DB8:1::1/64 Router(config-if)# ipv6 address FE80::1 link-local Router(config-if)# no shutdown ! Static IPv6 route Router(config)# ipv6 route 2001:DB8:2::/64 2001:DB8:12::2 ! Default IPv6 route Router(config)# ipv6 route ::/0 2001:DB8:12::2 ! Verify Router# show ipv6 interface brief Router# show ipv6 route

PART 2: ETHERNET SWITCHING

# Chapter 5: Ethernet & Switch Operation

Ethernet is the dominant Layer 2 technology for LANs. It defines how devices communicate over shared or switched networks using MAC addresses. Modern Ethernet uses full-duplex switched connections where each port is its own collision domain.

## 5.1 Ethernet Frame Structure

| Field | Size | Description |
| --- | --- | --- |
| Preamble | 7 bytes | 10101010... pattern — synchronizes receiver clock |
| SFD (Start Frame Delimiter) | 1 byte | 10101011 — marks start of frame |
| Destination MAC | 6 bytes | MAC address of receiving device |
| Source MAC | 6 bytes | MAC address of sending device |
| EtherType / Length | 2 bytes | Protocol type (0x0800=IPv4, 0x86DD=IPv6, 0x0806=ARP) or frame length |
| Data & Padding | 46–1500 bytes | Actual payload. Minimum 46B ensures collision detection at 10 Mbps. |
| FCS (Frame Check Sequence) | 4 bytes | CRC error detection — receiver recalculates and compares |

💡 NOTE MTU (Maximum Transmission Unit) = 1500 bytes for standard Ethernet. Jumbo frames extend this to 9000 bytes for data center environments.

## 5.2 MAC Addresses

A MAC (Media Access Control) address is a 48-bit (6-byte) hardware identifier burned into a NIC by the manufacturer. Written in hex, e.g., AA:BB:CC:DD:EE:FF.

| Portion | Bits | Description |
| --- | --- | --- |
| OUI (Organizationally Unique Identifier) | First 24 bits (3 bytes) | Identifies manufacturer (Cisco=00:1A:2B, etc.) |
| Device Identifier (NIC-specific) | Last 24 bits (3 bytes) | Unique per device — assigned by manufacturer |
| Bit 0 of byte 1 (I/G bit) | 1 bit | 0=Unicast, 1=Multicast/Broadcast |
| Bit 1 of byte 1 (U/L bit) | 1 bit | 0=Globally unique (OUI assigned), 1=Locally administered |

| MAC Address Type | Example | Description |
| --- | --- | --- |
| Unicast | 00:1A:2B:3C:4D:5E | Specific single device |
| Multicast | 01:00:5E:00:00:01 | Group of devices (bit 0 of first byte = 1) |
| Broadcast | FF:FF:FF:FF:FF:FF | All devices on the segment |

## 5.3 How a Switch Forwards Frames — Detailed

A switch builds a MAC Address Table (CAM table) to intelligently forward frames only to the correct destination port.

- Frame arrives: Switch reads the SOURCE MAC → records it in the MAC table with the incoming port + VLAN + timer.

- Switch looks up DESTINATION MAC in the MAC table.

- If found (unicast): Forwards frame only to that port — called known unicast forwarding.

- If NOT found (unicast): Floods frame to all ports in that VLAN except source — unknown unicast flood.

- If BROADCAST (FF:FF:FF:FF:FF:FF): Floods to all ports in VLAN except source.

- If MULTICAST: Floods unless IGMP snooping is enabled (see Chapter 20).

! View MAC address table Switch# show mac address-table Switch# show mac address-table dynamic Switch# show mac address-table address 0000.1111.2222 ! Clear MAC table Switch# clear mac address-table dynamic

## 5.4 Switch Port Modes & Duplex

| Setting | Description | Best Practice |
| --- | --- | --- |
| Full-Duplex | Send AND receive simultaneously — no collisions | Use always on switch-to-device links |
| Half-Duplex | Send OR receive at once — uses CSMA/CD | Legacy hubs only — avoid |
| Auto | Autonegotiates speed and duplex | Use on most edge ports |
| Hard-coded | Manually set speed and duplex | Use on uplinks / inter-switch links for consistency |

⚠️ WARNING DUPLEX MISMATCH: If one side is full-duplex and the other is half-duplex, performance degrades severely. Always hard-code speed/duplex on critical uplinks or use 'auto' consistently on both sides.

## 5.5 Switch Forwarding Methods

| Method | Latency | Error Checking | Description |
| --- | --- | --- | --- |
| Store-and-Forward | High | Full CRC check before forwarding | Receives entire frame, checks FCS, then forwards. Default on Cisco switches. |
| Cut-Through (Fast Forward) | Very Low | None | Begins forwarding after reading destination MAC only. Fastest but forwards corrupt frames. |
| Fragment-Free | Low | Reads first 64 bytes | Compromise — checks for collision fragments (< 64 bytes) then forwards. |

# Chapter 6: VLANs & Trunking

VLANs (Virtual Local Area Networks) logically segment a single physical switch into multiple isolated broadcast domains. Devices in different VLANs cannot communicate directly — they require a Layer 3 device (router or L3 switch).

## 6.1 VLAN Benefits

- Security: Isolate sensitive departments (Finance, HR) from general users

- Performance: Reduce broadcast domain size — fewer broadcasts per VLAN

- Cost Savings: Logical segmentation without physical hardware

- Flexibility: Move users to different VLANs without rewiring

- Simplify Management: Group devices by function, not location

## 6.2 VLAN ID Ranges

| VLAN Range | Type | Notes |
| --- | --- | --- |
| 1 | Default VLAN | All ports assigned here by default. Cannot be deleted. |
| 2 – 1001 | Normal VLANs | Standard user-configurable VLANs |
| 1002 – 1005 | Reserved | Legacy Token Ring / FDDI — cannot be used for Ethernet |
| 1006 – 4094 | Extended VLANs | Requires VTP transparent mode or VTP version 3. Stored only in running-config. |

## 6.3 VLAN Configuration

! Step 1: Create VLANs in VLAN database Switch(config)# vlan 10 Switch(config-vlan)# name SALES Switch(config)# vlan 20 Switch(config-vlan)# name ENGINEERING Switch(config)# vlan 30 Switch(config-vlan)# name MANAGEMENT ! Step 2: Assign Access Ports (end-user ports) Switch(config)# interface range fa0/1 - 10 Switch(config-if-range)# switchport mode access Switch(config-if-range)# switchport access vlan 10 Switch(config-if-range)# spanning-tree portfast ! Step 3: Configure Voice VLAN (IP phones) Switch(config-if)# switchport voice vlan 100 ! Verification Switch# show vlan brief Switch# show interfaces fa0/1 switchport

## 6.4 IEEE 802.1Q Trunking

A trunk port carries traffic for multiple VLANs between switches. 802.1Q inserts a 4-byte tag into the Ethernet frame to identify which VLAN the frame belongs to.

| 802.1Q Tag Field | Size | Description |
| --- | --- | --- |
| TPID (Tag Protocol ID) | 2 bytes | Always 0x8100 — identifies 802.1Q tagged frame |
| PCP (Priority Code Point) | 3 bits | QoS priority (CoS values 0–7) |
| DEI (Drop Eligible Indicator) | 1 bit | Indicates frame can be dropped under congestion |
| VID (VLAN Identifier) | 12 bits | VLAN number 0–4095 (0 and 4095 reserved) |

🔑 KEY CONCEPT Native VLAN: The one VLAN whose frames are sent UNTAGGED on a trunk. Default is VLAN 1. Both sides of a trunk must agree on the native VLAN or frames may be misrouted (VLAN hopping risk).

! Configure trunk port Switch(config)# interface GigabitEthernet0/1 Switch(config-if)# switchport trunk encapsulation dot1q Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk native vlan 999 Switch(config-if)# switchport trunk allowed vlan 10,20,30,100,999 ! DTP (Dynamic Trunking Protocol) — Cisco proprietary ! Disable DTP for security on access ports Switch(config-if)# switchport mode access Switch(config-if)# switchport nonegotiate ! Verify trunk Switch# show interfaces trunk Switch# show interfaces GigabitEthernet0/1 trunk

## 6.5 Inter-VLAN Routing

| Method | How | Pros | Cons |
| --- | --- | --- | --- |
| Router-on-a-Stick | Single router physical port with sub-interfaces per VLAN | Cost-effective; simple config | Bandwidth bottleneck; single router = SPOF |
| Layer 3 Switch with SVIs | Switch Virtual Interfaces (SVIs) per VLAN on L3 switch | High-speed ASIC routing; scalable | More expensive switch required |
| Separate Physical Links | One router interface per VLAN | Dedicated bandwidth | Not scalable; runs out of ports quickly |

! Router-on-a-Stick configuration Router(config)# interface GigabitEthernet0/0 Router(config-if)# no shutdown Router(config-if)# no ip address Router(config)# interface GigabitEthernet0/0.10 Router(config-subif)# encapsulation dot1Q 10 Router(config-subif)# ip address 192.168.10.1 255.255.255.0 Router(config)# interface GigabitEthernet0/0.20 Router(config-subif)# encapsulation dot1Q 20 Router(config-subif)# ip address 192.168.20.1 255.255.255.0 ! Layer 3 Switch SVI configuration Switch(config)# ip routing Switch(config)# interface vlan 10 Switch(config-if)# ip address 192.168.10.1 255.255.255.0 Switch(config-if)# no shutdown

# Chapter 7: Spanning Tree Protocol (STP)

STP (IEEE 802.1D) prevents Layer 2 loops in networks with redundant switch connections. Without STP, a broadcast storm would occur — frames looping endlessly, crashing the network. STP creates a loop-free logical topology while keeping redundant physical links available as backup.

## 7.1 STP Operation — Step by Step

- Elect Root Bridge: The switch with the lowest Bridge ID becomes Root Bridge.

- Elect Root Ports: Each non-root switch selects one Root Port (RP) — port closest to Root Bridge (lowest cost path).

- Elect Designated Ports: On each network segment, one switch port becomes Designated Port (DP) — the port sending BPDUs toward that segment.

- Block Remaining Ports: All non-Root, non-Designated ports are put in Blocking state.

🔑 KEY CONCEPT Bridge ID = Bridge Priority (16-bit, default 32768) + System ID Extension (12-bit VLAN ID) + MAC Address (48-bit tiebreaker). Lower Bridge ID wins Root Bridge election.

## 7.2 STP Port States & Timers

| State | Duration | Sends BPDUs? | Learns MACs? | Forwards Frames? | Description |
| --- | --- | --- | --- | --- | --- |
| Blocking | 20 sec (Max Age) | Receives only | No | No | Listens for BPDUs; no data forwarding |
| Listening | 15 sec (Fwd Delay) | Yes | No | No | Participates in active STP; no data |
| Learning | 15 sec (Fwd Delay) | Yes | Yes | No | Builds MAC table before forwarding |
| Forwarding | Until topology change | Yes | Yes | Yes | Normal operation — fully active |
| Disabled | N/A | No | No | No | Port manually shut down |

🔴 IMPORTANT Total STP convergence time = 50 seconds (20s Max Age + 15s Listening + 15s Learning). This is why RSTP was created.

## 7.3 STP Path Cost Values

| Link Speed | Short Mode Cost (802.1D-1998) | Long Mode Cost (802.1D-2004) |
| --- | --- | --- |
| 10 Mbps | 100 | 2,000,000 |
| 100 Mbps | 19 | 200,000 |
| 1 Gbps | 4 | 20,000 |
| 10 Gbps | 2 | 2,000 |
| 100 Gbps | 1 | 200 |

## 7.4 STP Variants

| Protocol | Standard | Instances | Convergence | Description |
| --- | --- | --- | --- | --- |
| STP (CST) | IEEE 802.1D-1998 | 1 (all VLANs) | ~50 seconds | Original STP — single tree for all VLANs |
| PVST+ | Cisco proprietary | 1 per VLAN | ~50 seconds | Per-VLAN STP — allows load balancing across VLANs |
| RSTP | IEEE 802.1w | 1 (all VLANs) | <1 second | Rapid STP — much faster convergence |
| Rapid PVST+ | Cisco (based on 802.1w) | 1 per VLAN | <1 second | Per-VLAN RSTP — Cisco default on modern switches |
| MSTP | IEEE 802.1s | Multiple instances | <1 second | Map multiple VLANs to fewer STP instances — scalable |

! Configure STP mode (Cisco default: rapid-pvst) Switch(config)# spanning-tree mode rapid-pvst ! Manipulate Root Bridge election Switch(config)# spanning-tree vlan 10 priority 4096 Switch(config)# spanning-tree vlan 10 root primary Switch(config)# spanning-tree vlan 20 root secondary ! PortFast — skip Listening/Learning on access ports Switch(config-if)# spanning-tree portfast ! BPDU Guard — shuts port if BPDU received (security) Switch(config-if)# spanning-tree bpduguard enable Switch(config)# spanning-tree portfast bpduguard default ! Root Guard — prevents downstream switch becoming Root Switch(config-if)# spanning-tree guard root ! Verify STP Switch# show spanning-tree vlan 10 Switch# show spanning-tree summary

# Chapter 8: EtherChannel (Link Aggregation)

EtherChannel bundles multiple physical links between switches into one logical link, providing increased bandwidth and redundancy. STP treats the bundle as a single link, preventing any of the physical links from being blocked.

## 8.1 EtherChannel Protocols

| Protocol | Standard | Modes | Description |
| --- | --- | --- | --- |
| LACP | IEEE 802.3ad (802.1ax) | Active / Passive | Open standard. Active initiates negotiation. Passive waits. At least one side must be Active. |
| PAgP | Cisco proprietary | Desirable / Auto | Cisco-only. Desirable initiates. Auto waits. At least one side must be Desirable. |
| Static (On) | N/A | On / On | No negotiation protocol. Both sides must be set to 'on'. Use with caution. |

! LACP EtherChannel Switch(config)# interface range GigabitEthernet0/1 - 2 Switch(config-if-range)# channel-protocol lacp Switch(config-if-range)# channel-group 1 mode active ! Configure the logical Port-Channel interface Switch(config)# interface Port-channel1 Switch(config-if)# switchport mode trunk Switch(config-if)# switchport trunk allowed vlan 10,20,30 ! Verify EtherChannel Switch# show etherchannel summary Switch# show etherchannel port-channel

⚠️ WARNING EtherChannel requirements: All member ports must have IDENTICAL configuration — same speed, duplex, VLAN membership, trunk settings. Mismatch causes the channel to fail.

# Chapter 9: Layer 2 Security

Layer 2 is often the weakest point in network security. Attackers can exploit MAC flooding, rogue DHCP servers, ARP spoofing, and VLAN hopping. Cisco provides specific features to mitigate each threat.

## 9.1 Port Security

Limits the number of MAC addresses allowed on a port to prevent MAC flooding attacks.

Switch(config-if)# switchport mode access Switch(config-if)# switchport port-security Switch(config-if)# switchport port-security maximum 3 Switch(config-if)# switchport port-security mac-address sticky Switch(config-if)# switchport port-security violation restrict Switch# show port-security interface fa0/1 Switch# show port-security address

| Violation Mode | Drops Frames? | Increments Counter? | Sends Syslog? | Disables Port? |
| --- | --- | --- | --- | --- |
| Protect | Yes | No | No | No |
| Restrict | Yes | Yes | Yes | No |
| Shutdown (default) | Yes | Yes | Yes | Yes — err-disabled |

! Recover err-disabled port Switch(config-if)# shutdown Switch(config-if)# no shutdown ! Or enable automatic recovery Switch(config)# errdisable recovery cause psecure-violation Switch(config)# errdisable recovery interval 300

## 9.2 DHCP Snooping

Prevents rogue DHCP servers by classifying ports as trusted (connected to real DHCP server) or untrusted (end-user ports). DHCP server messages arriving on untrusted ports are dropped.

Switch(config)# ip dhcp snooping Switch(config)# ip dhcp snooping vlan 10,20,30 ! Mark uplink to real DHCP server as trusted Switch(config-if)# ip dhcp snooping trust ! Rate-limit DHCP packets on untrusted ports Switch(config-if)# ip dhcp snooping limit rate 15 Switch# show ip dhcp snooping binding Switch# show ip dhcp snooping statistics

## 9.3 Dynamic ARP Inspection (DAI)

Validates ARP packets against the DHCP snooping binding table. Drops ARP packets with invalid IP-to-MAC mappings, preventing ARP spoofing / man-in-the-middle attacks. DHCP snooping must be enabled first.

Switch(config)# ip arp inspection vlan 10,20,30 ! Trust the uplink port (toward router/DHCP server) Switch(config-if)# ip arp inspection trust Switch# show ip arp inspection vlan 10

## 9.4 IP Source Guard

Validates IP packets against DHCP snooping binding table. Drops packets if source IP doesn't match the binding. Prevents IP address spoofing.

Switch(config-if)# ip verify source Switch(config-if)# ip verify source port-security

## 9.5 VLAN Hopping Attack & Prevention

VLAN hopping allows an attacker to access VLANs they are not authorized for. Two methods: Switch Spoofing (attacker pretends to be a switch using DTP) and Double Tagging (attacker crafts frames with two 802.1Q tags).

- Prevention: Disable DTP on all access ports → 'switchport nonegotiate'

- Prevention: Change the native VLAN to an unused VLAN (not VLAN 1)

- Prevention: Explicitly tag the native VLAN on trunks

- Prevention: Shut down all unused ports and assign to a dead VLAN

PART 3: ROUTING PROTOCOLS

# Chapter 10: Routing Fundamentals

Routing is the process of determining the best path for packets to travel from source to destination across multiple networks. Routers use routing tables to make forwarding decisions based on destination IP addresses.

## 10.1 Routing Table Components

| Component | Description | Example |
| --- | --- | --- |
| Network/Prefix | Destination network address | 192.168.1.0/24 |
| Administrative Distance (AD) | Trustworthiness of the route source. Lower = preferred. | OSPF=110, RIP=120 |
| Metric | Cost to reach destination within a protocol | OSPF cost, EIGRP composite metric |
| Next-Hop | IP address of next router toward destination | 10.0.0.2 |
| Outgoing Interface | Physical interface to send packet out | GigabitEthernet0/1 |
| Timestamp | How long ago the route was learned | 00:04:22 |

🔑 KEY CONCEPT Route Selection Order: 1) Longest prefix match (most specific route wins) 2) Lowest Administrative Distance (most trusted source) 3) Lowest metric (best path within same protocol)

## 10.2 Administrative Distance (AD) Values

| Route Source | AD | Notes |
| --- | --- | --- |
| Directly Connected | 0 | Most trusted — interface is up with IP assigned |
| Static Route | 1 | Manually configured — assumed trustworthy |
| EIGRP Summary | 5 | Cisco EIGRP auto-summary routes |
| BGP (External) | 20 | eBGP routes learned from different AS |
| EIGRP (Internal) | 90 | EIGRP within same AS |
| IGRP (legacy) | 100 | Outdated — no longer used |
| OSPF | 110 | Most common open-standard IGP |
| IS-IS | 115 | Used mainly by service providers |
| RIP v1/v2 | 120 | Old distance-vector protocol |
| EIGRP (External) | 170 | EIGRP routes redistributed from other protocols |
| BGP (Internal) | 200 | iBGP routes within same AS |
| Unknown / Unreachable | 255 | Never used for routing — route discarded |

## 10.3 Static Routing

! Standard static route Router(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2 ! Static route using exit interface (prefer next-hop for multi-access networks) Router(config)# ip route 192.168.2.0 255.255.255.0 GigabitEthernet0/1 ! Default route (gateway of last resort) Router(config)# ip route 0.0.0.0 0.0.0.0 10.0.0.1 ! Floating static route (higher AD = backup when dynamic route fails) Router(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2 130 ! Summarized static route Router(config)# ip route 192.168.0.0 255.255.252.0 10.0.0.2 Router# show ip route static

# Chapter 11: OSPF — Open Shortest Path First

OSPF (RFC 2328) is a link-state routing protocol that uses the Dijkstra Shortest Path First (SPF) algorithm. It's the most widely deployed interior gateway protocol (IGP) in enterprise networks. OSPFv2 handles IPv4; OSPFv3 handles IPv6.

## 11.1 OSPF Key Concepts

| Concept | Description |
| --- | --- |
| Link-State Database (LSDB) | Complete map of the network topology. All routers in an area have identical LSDBs. |
| LSA (Link-State Advertisement) | Contains topology information about a router. Flooded to all routers in the area. |
| Router ID (RID) | 32-bit identifier for each OSPF router. Must be unique. Used in LSA exchanges. |
| Area | Group of routers sharing the same LSDB. Area 0 = Backbone Area (mandatory). |
| DR/BDR | Designated Router / Backup DR — elected on multi-access networks to reduce OSPF traffic. |
| Hello Protocol | OSPF uses Hello packets to discover neighbors and maintain adjacencies. |
| Cost Metric | Based on interface bandwidth: Cost = 100,000,000 / bandwidth_bps. Lower = better. |

## 11.2 OSPF Neighbor Adjacency States

| State | Description |
| --- | --- |
| Down | No Hellos received yet. Initial state. |
| Init | Hello received from neighbor, but our router ID not in their Hello. |
| 2-Way | Bidirectional communication established. Router ID seen in neighbor's Hello. DR/BDR election happens here. |
| ExStart | Establish master/slave relationship to begin database exchange. Higher RID becomes master. |
| Exchange | DBD (Database Description) packets exchanged — summary of LSDBs compared. |
| Loading | LSR (Link-State Request) sent to get missing LSAs. LSU (Link-State Update) received in response. |
| Full | LSDBs synchronized — fully adjacent neighbors. Normal operational state. |

💡 NOTE On broadcast networks (Ethernet), non-DR/BDR routers only reach 2-Way state with each other, but reach Full state with the DR and BDR. On point-to-point links, all neighbors reach Full state.

## 11.3 OSPF Router Roles

| Router Type | Role |
| --- | --- |
| Internal Router | All interfaces in the same area |
| Backbone Router (BR) | At least one interface in Area 0 |
| Area Border Router (ABR) | Connects two or more OSPF areas. Maintains separate LSDB for each area. Summarizes routes between areas. |
| AS Boundary Router (ASBR) | Connects OSPF to external routing domains (e.g., redistributes BGP or EIGRP into OSPF). |
| DR (Designated Router) | Elected on multi-access segments. All routers form Full adjacency with DR. |
| BDR (Backup DR) | Takes over if DR fails. Monitors DR; pre-built relationships ready. |

## 11.4 OSPF Hello Timer Requirements

| Network Type | Hello Interval | Dead Interval | DR/BDR Election? | Example |
| --- | --- | --- | --- | --- |
| Broadcast (Ethernet) | 10 sec | 40 sec | Yes | GigabitEthernet |
| Point-to-Point | 10 sec | 40 sec | No | Serial, GRE tunnel |
| Non-Broadcast (NBMA) | 30 sec | 120 sec | Yes | Frame Relay (legacy) |
| Point-to-Multipoint | 30 sec | 120 sec | No | Hub-and-spoke WAN |

⚠️ WARNING OSPF neighbor adjacency will NOT form if: Hello/Dead timers mismatch, Area IDs differ, Authentication mismatch, Subnet masks differ (on broadcast), MTU mismatch, Stub flags mismatch.

## 11.5 OSPF Full Configuration

! Basic OSPF single-area configuration Router(config)# router ospf 1 Router(config-router)# router-id 1.1.1.1 Router(config-router)# network 192.168.1.0 0.0.0.255 area 0 Router(config-router)# network 10.0.0.0 0.0.0.3 area 0 Router(config-router)# passive-interface GigabitEthernet0/0 Router(config-router)# default-information originate ! Adjust cost manually on an interface Router(config-if)# ip ospf cost 50 ! Adjust reference bandwidth (set globally — all routers must match) Router(config-router)# auto-cost reference-bandwidth 10000 ! Configure DR priority (higher wins election, 0 = never DR) Router(config-if)# ip ospf priority 100 ! Multi-area OSPF — Area Border Router Router(config-router)# network 10.1.0.0 0.0.255.255 area 0 Router(config-router)# network 10.2.0.0 0.0.255.255 area 1 ! Area summarization on ABR Router(config-router)# area 1 range 10.2.0.0 255.255.0.0 ! Stub area — no external LSAs (reduces LSDB size) Router(config-router)# area 2 stub Router(config-router)# area 2 stub no-summary ! Authentication Router(config-router)# area 0 authentication message-digest Router(config-if)# ip ospf message-digest-key 1 md5 Cisco@123 ! Verify Router# show ip ospf Router# show ip ospf neighbor Router# show ip ospf database Router# show ip ospf interface GigabitEthernet0/0

# Chapter 12: EIGRP — Enhanced IGRP

EIGRP is a Cisco-developed advanced distance-vector routing protocol. It uses the DUAL (Diffusing Update Algorithm) algorithm to provide fast, loop-free convergence. EIGRP supports VLSM, uses partial updates, and supports multiple routed protocols.

## 12.1 EIGRP Key Concepts

| Term | Definition |
| --- | --- |
| FD (Feasible Distance) | Lowest total metric from local router to destination network. |
| RD (Reported Distance) | Metric reported by a neighbor to reach the destination (also called AD — Advertised Distance). |
| Successor | Primary route — the neighbor with the lowest FD. Installed in routing table. |
| Feasible Successor (FS) | Backup route — meets Feasibility Condition: RD < FD of Successor. Stored in topology table, instantly used if Successor fails. |
| Feasibility Condition | A neighbor can be an FS only if its RD is less than the current FD. Guarantees loop-free backup paths. |
| DUAL | Diffusing Update Algorithm — calculates loop-free paths. Runs when topology changes. |

## 12.2 EIGRP Composite Metric

EIGRP calculates metric using: Metric = [K1×Bandwidth + (K2×Bandwidth)/(256-Load) + K3×Delay] × [K5/(Reliability+K4)]

Default: K1=1, K2=0, K3=1, K4=0, K5=0  →  Simplified: Metric = 256 × (10^7/BW_Kbps + Delay_microsec/10)

| K Value | Factor | Default | Notes |
| --- | --- | --- | --- |
| K1 | Bandwidth | 1 (active) | Inverse of slowest bandwidth in path |
| K2 | Load | 0 (inactive) | Interface load 1-255 |
| K3 | Delay | 1 (active) | Sum of delays in the path (in 10 microsecond units) |
| K4 | Reliability | 0 (inactive) | Interface reliability 1-255 |
| K5 | MTU | 0 (inactive) | Minimum MTU along path |

! Basic EIGRP configuration (Classic mode) Router(config)# router eigrp 100 Router(config-router)# eigrp router-id 1.1.1.1 Router(config-router)# network 192.168.1.0 0.0.0.255 Router(config-router)# network 10.0.0.0 0.0.0.255 Router(config-router)# no auto-summary Router(config-router)# passive-interface GigabitEthernet0/0 ! Named mode EIGRP (modern approach) Router(config)# router eigrp COMPANY Router(config-router)# address-family ipv4 unicast autonomous-system 100 Router(config-router-af)# eigrp router-id 1.1.1.1 Router(config-router-af)# network 0.0.0.0 ! Summarize routes Router(config-if)# ip summary-address eigrp 100 192.168.0.0 255.255.252.0 ! Verify EIGRP Router# show ip eigrp neighbors Router# show ip eigrp topology Router# show ip eigrp topology all-links

# Chapter 13: BGP — Border Gateway Protocol

BGP (RFC 4271) is the routing protocol of the internet — the only Exterior Gateway Protocol (EGP) used today. It's a path-vector protocol that routes between Autonomous Systems (AS). Every AS has a unique AS number (ASN). BGP is essential for multi-homed internet connections and is also used within large enterprise and service provider networks.

## 13.1 BGP Types

| Type | Description | AD | TCP Port |
| --- | --- | --- | --- |
| eBGP (External BGP) | BGP between routers in DIFFERENT Autonomous Systems | 20 | 179 |
| iBGP (Internal BGP) | BGP between routers in the SAME Autonomous System | 200 | 179 |

🔑 KEY CONCEPT BGP uses TCP port 179 for all sessions. Unlike OSPF/EIGRP which use multicast, BGP requires manually configured neighbors. It does not use auto-discovery.

## 13.2 BGP Path Selection — Attribute Order

BGP selects the best path through a strict sequence of attributes. First attribute wins:

| Step | Attribute | Prefer |
| --- | --- | --- |
| 1 | Weight (Cisco proprietary) | Highest (local to router only) |
| 2 | Local Preference (LOCAL_PREF) | Highest (within AS — prefer exit point) |
| 3 | Locally Originated / Aggregated | Local routes preferred |
| 4 | AS Path Length | Shortest AS_PATH (fewer ASes to traverse) |
| 5 | Origin Type | IGP (i) > EGP (e) > Incomplete (?) |
| 6 | MED (Multi-Exit Discriminator) | Lowest (hint to neighboring AS for entry point) |
| 7 | eBGP over iBGP | eBGP preferred over iBGP learned routes |
| 8 | IGP Metric to Next-Hop | Lowest IGP cost to reach BGP next-hop |
| 9 | Oldest eBGP Route | Oldest (most stable) route preferred |
| 10 | Router ID | Lowest Router ID |
| 11 | Cluster List Length | Shortest cluster list |
| 12 | Neighbor IP Address | Lowest IP address of neighbor |

✅ TIP Memory: 'We Love Oranges As Oranges Mean Pure Refreshment' (Weight, Local Pref, Originate, AS Path, Origin, MED, Prefer eBGP, Routing metric...)

! Basic BGP configuration Router(config)# router bgp 65001 Router(config-router)# bgp router-id 1.1.1.1 Router(config-router)# neighbor 203.0.113.2 remote-as 65002 Router(config-router)# neighbor 203.0.113.2 description ISP-A ! Advertise networks into BGP Router(config-router)# network 198.51.100.0 mask 255.255.255.0 ! iBGP peer Router(config-router)# neighbor 10.0.0.2 remote-as 65001 Router(config-router)# neighbor 10.0.0.2 update-source Loopback0 Router(config-router)# neighbor 10.0.0.2 next-hop-self ! Default route to iBGP peers Router(config-router)# default-information originate ! Verify BGP Router# show bgp summary Router# show bgp ipv4 unicast Router# show bgp neighbors 203.0.113.2

# Chapter 14: Route Redistribution & Policy-Based Routing

Route redistribution moves routing information between different routing protocols. Policy-Based Routing (PBR) overrides normal routing decisions based on criteria like source IP, protocol, or packet size.

## 14.1 Route Redistribution

! Redistribute OSPF into EIGRP Router(config)# router eigrp 100 Router(config-router)# redistribute ospf 1 metric 10000 100 255 1 1500 ! Redistribute EIGRP into OSPF Router(config)# router ospf 1 Router(config-router)# redistribute eigrp 100 subnets metric 50 metric-type 2 ! Redistribute static routes into OSPF Router(config-router)# redistribute static subnets ! Use route-map to filter during redistribution Router(config)# route-map FILTER_OSPF permit 10 Router(config-route-map)# match ip address prefix-list ALLOWED Router(config-router)# redistribute ospf 1 route-map FILTER_OSPF

⚠️ WARNING Mutual redistribution (two routers each redistributing between the same two protocols) can create routing loops. Always use route-maps, tags, and administrative distance manipulation to prevent this.

## 14.2 Policy-Based Routing (PBR)

! PBR — route traffic from 192.168.1.0/24 via ISP-B instead of default Router(config)# access-list 101 permit ip 192.168.1.0 0.0.0.255 any Router(config)# route-map PBR_ISP_B permit 10 Router(config-route-map)# match ip address 101 Router(config-route-map)# set ip next-hop 203.0.113.6 ! Apply PBR on inbound interface (where traffic enters) Router(config-if)# ip policy route-map PBR_ISP_B Router# show route-map Router# debug ip policy

# Chapter 15: First Hop Redundancy Protocols

FHRPs provide default gateway redundancy. If the active gateway router fails, another router takes over automatically without requiring end-user reconfiguration. They use a Virtual IP (VIP) address that clients use as their default gateway.

## 15.1 FHRP Comparison

| Protocol | Standard | Virtual IP? | Virtual MAC? | Load Balance? | Election |
| --- | --- | --- | --- | --- | --- |
| HSRP v1 | Cisco | Yes | 0000.0C07.ACxx | No (active/standby only) | Priority (100 def); highest IP tiebreak |
| HSRP v2 | Cisco | Yes | 0000.0C9F.Fxxx | No (active/standby only) | Priority (100 def); group 0–255 ext |
| VRRP | IEEE 802.3 | Yes | 0000.5E00.01xx | No (master/backup) | Priority (100 def); actual IP owner = 255 |
| GLBP | Cisco | Yes | 0007.B400.xxyy | Yes — up to 4 AVFs | AVG elected (priority); load shares across all |

! HSRP v2 configuration Router(config-if)# standby version 2 Router(config-if)# standby 1 ip 192.168.1.1 Router(config-if)# standby 1 priority 110 Router(config-if)# standby 1 preempt Router(config-if)# standby 1 authentication md5 key-string Cisco@1 Router(config-if)# standby 1 track GigabitEthernet0/1 20 ! GLBP configuration Router(config-if)# glbp 1 ip 192.168.1.1 Router(config-if)# glbp 1 priority 150 Router(config-if)# glbp 1 preempt Router(config-if)# glbp 1 load-balancing round-robin Router# show standby brief Router# show glbp brief

PART 4: NETWORK SERVICES & SECURITY

# Chapter 16: Access Control Lists (ACLs)

ACLs are ordered rule sets that permit or deny traffic based on defined criteria. They are applied on router/switch interfaces and processed top-to-bottom — the first matching rule is applied, and the rest are skipped. An implicit 'deny all' exists at the end of every ACL.

## 16.1 ACL Types

| Type | Number Range | Matches On | Placement |
| --- | --- | --- | --- |
| Standard IPv4 (numbered) | 1–99, 1300–1999 | Source IP only | Close to destination (to avoid blocking too broadly) |
| Extended IPv4 (numbered) | 100–199, 2000–2699 | Source IP, Destination IP, Protocol, Port | Close to source (to stop traffic early) |
| Standard IPv4 (named) | Any name | Source IP only | Close to destination |
| Extended IPv4 (named) | Any name | Source IP, Destination IP, Protocol, Port | Close to source |
| IPv6 ACL | Named only | Source/Destination IPv6 address, Protocol, Port | Close to source |

🔑 KEY CONCEPT Wildcard Mask Logic: 0 = check this bit, 1 = ignore this bit. Opposite of subnet mask. • host 192.168.1.5 = 192.168.1.5 0.0.0.0 • 192.168.1.0/24 = 192.168.1.0 0.0.0.255 • any = 0.0.0.0 255.255.255.255

## 16.2 ACL Configuration

! Standard numbered ACL Router(config)# access-list 10 permit 192.168.1.0 0.0.0.255 Router(config)# access-list 10 deny any ! Extended numbered ACL Router(config)# access-list 110 deny tcp 192.168.1.0 0.0.0.255 host 10.0.0.5 eq 23 Router(config)# access-list 110 deny tcp 192.168.1.0 0.0.0.255 host 10.0.0.5 eq 21 Router(config)# access-list 110 permit ip any any ! Named extended ACL (preferred — human readable, editable) Router(config)# ip access-list extended BLOCK_TELNET Router(config-ext-nacl)# 10 deny tcp any any eq 23 Router(config-ext-nacl)# 20 deny tcp any any eq 21 Router(config-ext-nacl)# 30 permit ip any any ! Apply ACL to interface Router(config-if)# ip access-group 110 in Router(config-if)# ip access-group BLOCK_TELNET out ! Apply ACL to VTY lines (restrict SSH/Telnet access) Router(config)# line vty 0 4 Router(config-line)# access-class 10 in ! Verify ACLs Router# show access-lists Router# show ip interface GigabitEthernet0/0 ! IPv6 ACL Router(config)# ipv6 access-list BLOCK_V6_TELNET Router(config-ipv6-acl)# deny tcp any any eq 23 Router(config-ipv6-acl)# permit ipv6 any any Router(config-if)# ipv6 traffic-filter BLOCK_V6_TELNET in

# Chapter 17: NAT & PAT

NAT (Network Address Translation) maps private IP addresses to public IPs, enabling private hosts to access the internet. It also provides a layer of security by hiding internal addressing. PAT (Port Address Translation) extends NAT to allow many devices to share one public IP using unique port numbers.

## 17.1 NAT Terminology

| Term | Description |
| --- | --- |
| Inside Local | Private IP address of an internal device (before translation) |
| Inside Global | Public IP assigned to internal device by NAT (after translation — seen by internet) |
| Outside Local | IP address of external device as seen from inside the network |
| Outside Global | Actual public IP of the external device |
| NAT Table | Maintains mappings between inside local and inside global addresses |

## 17.2 NAT Types & Configuration

| NAT Type | Mapping | Use Case | Key Command |
| --- | --- | --- | --- |
| Static NAT | One private IP ↔ One public IP (permanent) | Web server, mail server needing fixed public IP | ip nat inside source static |
| Dynamic NAT | Private IP ↔ Pool of public IPs (first available) | Many users, limited public IPs | ip nat pool + ip nat inside source list |
| PAT / Overload | Many private IPs ↔ One public IP (uses ports) | Home/office — most common today | ip nat inside source list ... overload |

! Mark interfaces Router(config-if)# ip nat inside Router(config-if)# ip nat outside ! Static NAT Router(config)# ip nat inside source static 192.168.1.10 203.0.113.5 ! Dynamic NAT with pool Router(config)# ip nat pool PUBLIC_POOL 203.0.113.10 203.0.113.20 netmask 255.255.255.0 Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255 Router(config)# ip nat inside source list 1 pool PUBLIC_POOL ! PAT (overload) — most common Router(config)# access-list 1 permit 192.168.0.0 0.0.255.255 Router(config)# ip nat inside source list 1 interface GigabitEthernet0/1 overload ! Verify NAT Router# show ip nat translations Router# show ip nat statistics Router# debug ip nat

# Chapter 18: DHCP, DNS, and NTP

## 18.1 DHCP — Dynamic Host Configuration Protocol

DHCP automatically provides IP configuration to clients: IP address, subnet mask, default gateway, DNS server, and lease time. The DORA process: Discover → Offer → Request → Acknowledge.

| DORA Step | From → To | Type | Description |
| --- | --- | --- | --- |
| Discover | Client → Broadcast | UDP Broadcast (src:0.0.0.0, dst:255.255.255.255) | Client broadcasts to find DHCP servers |
| Offer | Server → Broadcast/Unicast | UDP from server | Server offers an IP address and config |
| Request | Client → Broadcast | UDP Broadcast | Client requests the offered IP (also notifies other servers) |
| Acknowledge | Server → Client | UDP from server | Server confirms — client applies configuration |

! Configure router as DHCP server Router(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.20 Router(config)# ip dhcp pool OFFICE_LAN Router(dhcp-config)# network 192.168.1.0 255.255.255.0 Router(dhcp-config)# default-router 192.168.1.1 Router(dhcp-config)# dns-server 8.8.8.8 8.8.4.4 Router(dhcp-config)# domain-name company.local Router(dhcp-config)# lease 7 ! DHCP Relay — forward broadcasts to remote DHCP server Router(config-if)# ip helper-address 10.0.0.100 ! Verify DHCP Router# show ip dhcp binding Router# show ip dhcp pool Router# show ip dhcp conflict

## 18.2 DNS — Domain Name System

DNS resolves human-readable domain names (www.google.com) to IP addresses. It uses a hierarchical structure: Root → TLD (.com, .org) → Authoritative nameservers → Local DNS cache.

| DNS Record Type | Description | Example |
| --- | --- | --- |
| A | IPv4 address for a hostname | www.example.com → 93.184.216.34 |
| AAAA | IPv6 address for a hostname | www.example.com → 2606:2800:: |
| CNAME | Alias — points to another hostname | ftp.example.com → www.example.com |
| MX | Mail server for a domain | example.com → mail.example.com (priority 10) |
| PTR | Reverse lookup — IP to hostname | 93.184.216.34 → www.example.com |
| NS | Authoritative nameserver for domain | example.com NS ns1.example.com |
| TXT | Text records — used for SPF, DKIM, verification | v=spf1 include:example.com ~all |
| SOA | Start of Authority — zone info | Primary NS, admin email, serial, TTL values |

! Configure DNS on Cisco device Router(config)# ip domain-name company.local Router(config)# ip name-server 8.8.8.8 Router(config)# ip name-server 8.8.4.4 Router(config)# ip domain-lookup ! Static DNS entry Router(config)# ip host SERVER1 192.168.1.100

## 18.3 NTP — Network Time Protocol

NTP synchronizes clocks across network devices. Accurate time is critical for log correlation, certificate validation, Kerberos authentication, and routing protocol operations. NTP uses a stratum hierarchy — Stratum 0 (atomic clock) → Stratum 1 (primary server) → Stratum 2 (clients).

| Stratum Level | Description |
| --- | --- |
| Stratum 0 | Atomic clocks, GPS receivers — not directly on network |
| Stratum 1 | Directly connected to Stratum 0 — highest accuracy NTP servers |
| Stratum 2 | Syncs from Stratum 1. Typical enterprise NTP server. |
| Stratum 3–15 | Each level syncs from the one above. Accuracy decreases. |
| Stratum 16 | Unsynchronized — NTP not working |

! Configure NTP client Router(config)# ntp server 216.239.35.0 prefer Router(config)# ntp server 216.239.35.4 ! Configure as NTP master (if no upstream server available) Router(config)# ntp master 3 ! NTP authentication Router(config)# ntp authenticate Router(config)# ntp authentication-key 1 md5 NTPkey@1 Router(config)# ntp trusted-key 1 Router(config)# ntp server 10.0.0.1 key 1 ! Verify Router# show ntp status Router# show ntp associations

# Chapter 19: Network Security — Device Hardening & AAA

## 19.1 Cisco IOS Hardening

! Secure privileged EXEC access Router(config)# enable secret StrongPass@2024 Router(config)# service password-encryption ! Secure console port Router(config)# line console 0 Router(config-line)# exec-timeout 10 0 Router(config-line)# logging synchronous Router(config-line)# login local ! Secure VTY lines — SSH only Router(config)# line vty 0 4 Router(config-line)# transport input ssh Router(config-line)# exec-timeout 10 0 Router(config-line)# login local Router(config-line)# access-class 10 in ! Configure SSH v2 Router(config)# ip domain-name corp.local Router(config)# crypto key generate rsa modulus 2048 Router(config)# ip ssh version 2 Router(config)# ip ssh time-out 60 Router(config)# ip ssh authentication-retries 3 ! Local user accounts Router(config)# username admin privilege 15 secret Admin@pass1 Router(config)# username helpdesk privilege 5 secret Help@desk2 ! Disable unused services Router(config)# no ip http server Router(config)# no ip http secure-server Router(config)# no cdp run Router(config)# no ip finger Router(config)# no service tcp-small-servers Router(config)# no service udp-small-servers ! Login security — slow brute force attacks Router(config)# login block-for 120 attempts 3 within 60 Router(config)# login delay 3 Router(config)# security passwords min-length 10 ! Legal banner Router(config)# banner motd # Authorized Access Only! #

## 19.2 AAA — Authentication, Authorization, Accounting

| Function | Description | Example |
| --- | --- | --- |
| Authentication | Verifies who you are — identity | Username + password, certificate, MFA |
| Authorization | Determines what you can do — permissions | Privilege levels, commands allowed |
| Accounting | Records what you did — audit trail | Login time, commands executed, bytes transferred |

| AAA Protocol | Port | Encryption | Description |
| --- | --- | --- | --- |
| TACACS+ | TCP 49 | Encrypts ENTIRE packet body | Cisco proprietary. Separates AAA functions. Preferred for device administration. |
| RADIUS | UDP 1812/1813 | Encrypts password only | Open standard. Combines Authentication + Authorization. Better for network access (802.1X, VPN). |

! Configure AAA with TACACS+ Router(config)# aaa new-model Router(config)# tacacs-server host 10.0.0.50 key TacKey@1 Router(config)# aaa authentication login default group tacacs+ local Router(config)# aaa authorization exec default group tacacs+ local Router(config)# aaa accounting exec default start-stop group tacacs+

# Chapter 20: Quality of Service (QoS)

QoS manages network traffic to ensure critical applications (voice, video, mission-critical data) receive appropriate bandwidth, low latency, and low jitter. QoS is essential when bandwidth is insufficient for all traffic simultaneously.

## 20.1 QoS Terminology

| Term | Definition |
| --- | --- |
| Bandwidth | Capacity of a link — maximum data rate (bps) |
| Delay (Latency) | Time for packet to travel from source to destination |
| Jitter | Variation in delay between packets — critical for VoIP (should be < 30ms) |
| Packet Loss | Percentage of packets dropped — should be near 0% for voice/video |
| DSCP | Differentiated Services Code Point — 6-bit field in IP header for QoS marking |
| CoS | Class of Service — 3-bit field in 802.1Q frame header |
| Queue | Buffer where packets wait before being forwarded |

## 20.2 QoS Models

| Model | Description | Mechanism |
| --- | --- | --- |
| Best Effort | Default — no QoS. All packets treated equally. FIFO queue. | No configuration needed |
| IntServ | Integrated Services — per-flow reservation using RSVP. Guarantees for individual flows. | RSVP (Resource Reservation Protocol) — complex, not scalable |
| DiffServ | Differentiated Services — classify traffic into classes, apply policies per class. Scalable. | DSCP marking, CBWFQ, LLQ — industry standard |

## 20.3 DSCP & CoS Values

| Traffic Type | DSCP Name | DSCP Value | CoS | PHB |
| --- | --- | --- | --- | --- |
| VoIP (voice bearer) | EF (Expedited Forwarding) | 46 (101110) | 5 | Low latency, guaranteed BW |
| Video conferencing | AF41 | 34 (100010) | 4 | Assured forwarding |
| Call signaling | CS3 | 24 (011000) | 3 | Class selector |
| Business critical apps | AF31 | 26 (011010) | 3 | Assured forwarding |
| Transactional data | AF21 | 18 (010010) | 2 | Assured forwarding |
| Bulk data (FTP, backup) | AF11 | 10 (001010) | 1 | Assured forwarding |
| Network management | CS6 | 48 (110000) | 6 | Routing protocols, SNMP |
| Scavenger (P2P, junk) | CS1 | 8 (001000) | 1 | Best effort or below |
| Default (best effort) | BE / DF | 0 (000000) | 0 | Default forwarding |

## 20.4 QoS Tools

| QoS Tool | Purpose | Description |
| --- | --- | --- |
| Classification | Identify traffic | match ip dscp, match protocol, match access-group |
| Marking | Label traffic for downstream | set dscp ef, set cos 5 — done as close to source as possible |
| CBWFQ | Bandwidth allocation | Class-Based Weighted Fair Queuing — guarantees minimum BW per class |
| LLQ | Voice priority | Low Latency Queue — strict priority queue for voice. EF traffic jumps to head of queue. |
| Traffic Shaping | Smooth bursts | Buffers excess traffic and releases at configured rate. Delay but don't drop. |
| Traffic Policing | Enforce rate limit | Drops or re-marks excess traffic. Immediate response to over-limit traffic. |
| WRED | Congestion avoidance | Weighted Random Early Detection — randomly drops low-priority packets before queue fills |
| LFI | Link fragmentation | Fragments large packets on slow WAN links to reduce serialization delay for voice |

! MQC (Modular QoS CLI) — industry-standard approach ! Step 1: Define class-maps (traffic classification) Router(config)# class-map match-any VOICE Router(config-cmap)# match ip dscp ef Router(config)# class-map match-any VIDEO Router(config-cmap)# match ip dscp af41 Router(config)# class-map match-any CRITICAL Router(config-cmap)# match ip dscp af31 af32 af33 ! Step 2: Define policy-map (what to do with each class) Router(config)# policy-map WAN_QOS Router(config-pmap)# class VOICE Router(config-pmap-c)# priority 2000 Router(config-pmap)# class VIDEO Router(config-pmap-c)# bandwidth percent 30 Router(config-pmap)# class CRITICAL Router(config-pmap-c)# bandwidth percent 20 Router(config-pmap)# class class-default Router(config-pmap-c)# fair-queue ! Step 3: Apply service-policy to interface Router(config-if)# service-policy output WAN_QOS

PART 5: WAN, VPN & WIRELESS

# Chapter 21: WAN Technologies

Wide Area Networks connect geographically dispersed locations. WAN technologies range from legacy TDM circuits to modern broadband and software-defined solutions. Choosing the right WAN technology depends on bandwidth requirements, budget, reliability, and latency needs.

## 21.1 WAN Technology Comparison

| Technology | Type | Speed | Reliability | Cost | Use Case |
| --- | --- | --- | --- | --- | --- |
| Leased Line (T1/E1) | Dedicated TDM | 1.5/2 Mbps | High | High | Branch office with strict SLA requirements |
| T3/E3 | Dedicated TDM | 45/34 Mbps | High | Very High | Large enterprise aggregation |
| DSL (ADSL2+) | Shared xDSL | 24 Mbps down / 3 Mbps up | Medium | Low | Small offices, home internet |
| VDSL2 / Fiber+DSL | Shared xDSL | 100–200 Mbps | Medium-High | Medium | Medium business broadband |
| Cable (DOCSIS 3.1) | Shared HFC | Up to 10 Gbps down | Medium | Low-Med | SMB internet access |
| Metro Ethernet | Dedicated fiber | 10 Mbps–10 Gbps | High | Medium-High | Campus/metro area enterprise connections |
| MPLS | Provider-switched | Varies (2M–10G) | Very High | High | Enterprise multi-site; QoS-sensitive traffic |
| SD-WAN | Overlay software | Depends on underlay | Very High | Medium | Multi-site with multiple underlay links; cloud access |
| 4G LTE | Cellular wireless | 10–150 Mbps | Medium | Medium | Remote sites, WAN backup, IoT |
| 5G NR | Cellular wireless | 100 Mbps–10 Gbps | Medium-High | Medium-High | High-bandwidth wireless WAN, urban areas |
| Satellite (GEO) | Satellite | 12–100 Mbps | Medium | High | Remote/maritime — high latency (600ms RTT) |
| Satellite (LEO - Starlink) | Satellite | 50–500 Mbps | Medium-High | Medium | Remote areas — low latency (20-40ms) |

## 21.2 MPLS Architecture

MPLS (Multiprotocol Label Switching) adds short fixed-length labels to packets. Routers make forwarding decisions based on labels (not IP lookups), making it fast and capable of transporting multiple protocols.

| Component | Role |
| --- | --- |
| Label Edge Router (LER) / PE Router | Provider Edge — adds (push) or removes (pop) labels at network edges. Connects customer sites to MPLS network. |
| Label Switching Router (LSR) / P Router | Provider Core — forwards packets based on labels only. Never sees customer IP addresses. |
| CE Router | Customer Edge — customer's router. Connects to PE. Usually runs BGP or static routing with PE. |
| LSP (Label Switched Path) | The end-to-end path through MPLS network determined by LDP or RSVP-TE. |
| LDP (Label Distribution Protocol) | Distributes label-to-FEC bindings between MPLS routers. Port UDP/TCP 646. |
| MPLS VPN (L3VPN) | Customer VRFs keep traffic isolated. PE routers use MP-BGP to exchange customer routes. |
| MPLS TE (Traffic Engineering) | Uses RSVP-TE to establish LSPs with bandwidth guarantees along specific paths. |

💡 NOTE MPLS label: 20-bit label + 3-bit EXP (QoS) + 1-bit Stack (S) bit + 8-bit TTL. Total = 32 bits (4 bytes). Label stack allows multiple labels (e.g., TE + VPN).

# Chapter 22: VPN Technologies

VPNs create encrypted tunnels over untrusted networks (like the internet), allowing secure communication between sites or individual users and a corporate network.

## 22.1 VPN Types

| VPN Type | Description | Use Case |
| --- | --- | --- |
| Site-to-Site IPsec VPN | Permanent encrypted tunnel between two routers over internet | HQ to branch office |
| Remote Access VPN | Individual users connect to corporate network using VPN client | Work-from-home, travelers |
| SSL/TLS VPN | Browser-based or thin client VPN using HTTPS port 443 | Clientless access; firewall-friendly |
| DMVPN | Dynamic Multipoint VPN — Hub-and-spoke with dynamic spoke-to-spoke tunnels | Many branch offices with direct communication |
| FlexVPN | Cisco's modern unified VPN framework using IKEv2 — flexible & scalable | Enterprise environments |
| GRE Tunnel | Generic Routing Encapsulation — IP-in-IP. NOT encrypted by default. | Carry multicast, routing protocols over WAN |
| GRE over IPsec | GRE tunnel protected by IPsec encryption | Routing protocols + encryption over WAN |

## 22.2 IPsec Fundamentals

| IPsec Component | Description |
| --- | --- |
| IKE Phase 1 | Negotiates ISAKMP SA. Establishes secure authenticated channel. Uses Diffie-Hellman key exchange. Produces IKE SA. |
| IKE Phase 2 | Negotiates IPsec SA (AH or ESP). Uses existing IKE SA. Produces IPsec SAs for actual data protection. |
| AH (Authentication Header) | Provides integrity and authentication. NO encryption. Protocol 51. |
| ESP (Encapsulating Security Payload) | Provides integrity, authentication, AND encryption. Protocol 50. Most commonly used. |
| Transport Mode | Protects only payload — original IP header intact. Used for host-to-host. |
| Tunnel Mode | Encapsulates entire original packet in new IP packet. Used for site-to-site VPN. |
| IKEv1 | Original IKE — 2 phases, more overhead. Widely deployed legacy. |
| IKEv2 | Modern standard — simpler, faster, more secure. Supports EAP and MOBIKE. |

! Site-to-Site IPsec VPN (IKEv1) ! Phase 1 policy Router(config)# crypto isakmp policy 10 Router(config-isakmp)# encryption aes 256 Router(config-isakmp)# hash sha256 Router(config-isakmp)# authentication pre-share Router(config-isakmp)# group 14 Router(config-isakmp)# lifetime 86400 Router(config)# crypto isakmp key VPNkey@2024 address 203.0.113.2 ! Phase 2 transform set Router(config)# crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac Router(cfg-crypto-trans)# mode tunnel ! Crypto map Router(config)# crypto map CMAP 10 ipsec-isakmp Router(config-crypto-map)# set peer 203.0.113.2 Router(config-crypto-map)# set transform-set TSET Router(config-crypto-map)# match address INTERESTING_TRAFFIC Router(config-if)# crypto map CMAP ! Verify IPsec VPN Router# show crypto isakmp sa Router# show crypto ipsec sa

# Chapter 23: Wireless Networking

Wireless LANs (WLANs) provide network access without physical cables using radio frequency (RF) signals. IEEE 802.11 defines the standards for wireless networking. Understanding RF principles, standards, security, and architecture is essential for modern network engineers.

## 23.1 IEEE 802.11 Standards

| Standard | Wi-Fi Name | Year | Frequency | Max Throughput | Key Features |
| --- | --- | --- | --- | --- | --- |
| 802.11a | — | 1999 | 5 GHz | 54 Mbps | OFDM; less interference; short range |
| 802.11b | — | 1999 | 2.4 GHz | 11 Mbps | DSSS; first widely adopted; crowded band |
| 802.11g | — | 2003 | 2.4 GHz | 54 Mbps | OFDM; backward compatible with 802.11b |
| 802.11n | Wi-Fi 4 | 2009 | 2.4 / 5 GHz | 600 Mbps | MIMO (4×4); channel bonding (40 MHz) |
| 802.11ac | Wi-Fi 5 | 2013 | 5 GHz only | 3.5 Gbps | MU-MIMO; 160 MHz channels; beamforming |
| 802.11ax | Wi-Fi 6 | 2019 | 2.4 / 5 / 6 GHz | 9.6 Gbps | OFDMA; BSS coloring; TWT; dense environments |
| 802.11be | Wi-Fi 7 | 2024 | 2.4/5/6 GHz | 46 Gbps | 320 MHz channels; multi-link operation (MLO) |

## 23.2 2.4 GHz vs 5 GHz Comparison

| Feature | 2.4 GHz | 5 GHz | 6 GHz (Wi-Fi 6E) |
| --- | --- | --- | --- |
| Range | Longer (signal penetrates walls better) | Shorter | Shortest |
| Speed | Lower | Higher | Highest |
| Channels (non-overlap) | 3 (1, 6, 11) | 24 (20 MHz) up to 45 | 59 channels (20 MHz) |
| Interference | High (microwaves, Bluetooth, neighbors) | Lower | Very Low (new band) |
| Device density | Poor in crowded areas | Better | Best |

## 23.3 Wireless Security Standards

| Standard | Year | Encryption | Authentication | Vulnerability |
| --- | --- | --- | --- | --- |
| WEP | 1997 | RC4 (40/104-bit) | Shared key / Open | Broken — IV reuse attack. Never use. |
| WPA (TKIP) | 2003 | RC4 + TKIP | PSK or 802.1X/EAP | Weak — interim standard |
| WPA2-Personal | 2004 | AES-CCMP (128-bit) | PSK (passphrase) | Vulnerable to PMKID & dictionary attacks |
| WPA2-Enterprise | 2004 | AES-CCMP (128-bit) | 802.1X / EAP + RADIUS | Strong — use for business networks |
| WPA3-Personal | 2018 | AES-GCMP (192-bit) | SAE (Simultaneous Authentication of Equals) | Resistant to offline dictionary attacks |
| WPA3-Enterprise | 2018 | AES-GCMP (192-bit) | 802.1X/EAP + RADIUS | Strongest — 192-bit security mode |
| OWE (Enhanced Open) | 2018 | Opportunistic encryption | None (open network) | Encrypts open Wi-Fi without passwords |

🔴 IMPORTANT Always use WPA2-Enterprise or WPA3 for business networks. WEP is completely broken — crackable in under 60 seconds. WPA with TKIP is also deprecated.

## 23.4 Wireless Deployment Architectures

| Architecture | Description | Pros | Cons |
| --- | --- | --- | --- |
| Autonomous AP | Each AP independently configured. Standalone mode. | Simple for small deployments | Doesn't scale; no central management; roaming issues |
| Controller-Based (Centralized) | APs (lightweight) controlled by WLC (Wireless LAN Controller) | Centralized config & monitoring; seamless roaming; RF management | WLC is SPOF unless redundant; more complex |
| Cisco FlexConnect | AP can locally switch traffic if WLC unreachable (hybrid) | Branch resilience during WAN failure | More complex configuration |
| Cloud-Managed (Meraki, Catalyst Center) | APs managed via cloud portal | Easy deployment; zero-touch provisioning; anywhere access | Requires internet for management; subscription cost |

! CAPWAP — Control and Provisioning of Wireless Access Points ! Protocol used between Lightweight AP (LAP) and WLC ! CAPWAP Control: UDP 5246 (management) ! CAPWAP Data: UDP 5247 (data tunneling) ! AP joins WLC via CAPWAP discovery: ! 1. DHCP option 43 (WLC IP list) ! 2. DNS: cisco-capwap-controller.domain.com ! 3. Layer 3 broadcast ! 4. Previously remembered WLC

PART 6: CCNP ADVANCED TOPICS

# Chapter 24: SNMP, Syslog & Network Monitoring

## 24.1 SNMP — Simple Network Management Protocol

SNMP provides a framework for monitoring and managing network devices. It uses a manager-agent model — the NMS (Network Management Station) queries agents on devices to collect statistics and can send commands to configure devices.

| Component | Description |
| --- | --- |
| NMS (Manager) | Central monitoring system (SolarWinds, PRTG, Nagios, Cisco Prime, etc.) |
| Agent | Software running on network device that collects and reports data |
| MIB (Management Information Base) | Database of objects that can be monitored/managed. Hierarchical OID structure. |
| OID (Object Identifier) | Unique identifier for each manageable object (e.g., .1.3.6.1.2.1.1.1 = sysDescr) |

| SNMP Version | Security | Authentication | Encryption | Recommendation |
| --- | --- | --- | --- | --- |
| SNMPv1 | None | Community string (cleartext) | None | Legacy only — avoid |
| SNMPv2c | None | Community string (cleartext) | None | Common but insecure |
| SNMPv3 | Strong | MD5 or SHA username/password | DES or AES | Use in production — always |

| SNMP Operation | Port | Direction | Description |
| --- | --- | --- | --- |
| Get | UDP 161 | NMS → Agent | Request specific OID value |
| GetNext | UDP 161 | NMS → Agent | Request next OID in MIB tree |
| GetBulk | UDP 161 | NMS → Agent | Request multiple OIDs efficiently |
| Set | UDP 161 | NMS → Agent | Write a value to device |
| Trap | UDP 162 | Agent → NMS | Unsolicited alert from agent (no confirmation) |
| Inform | UDP 162 | Agent → NMS | Like Trap but NMS sends acknowledgment |

! SNMPv3 configuration Router(config)# snmp-server group ADMIN v3 priv Router(config)# snmp-server user SNMPUSER ADMIN v3 auth sha Auth@Pass1 priv aes 128 Priv@Key1 Router(config)# snmp-server host 10.0.0.200 version 3 priv SNMPUSER Router(config)# snmp-server enable traps Router(config)# snmp-server location ServerRoom-Rack3 Router(config)# snmp-server contact noc@company.com

## 24.2 Syslog

Syslog is a standard protocol for sending log messages from network devices to a central syslog server. Every log message has a severity level (0–7) and a facility code identifying the source subsystem.

| Level | Keyword | Description | Example Message |
| --- | --- | --- | --- |
| 0 | Emergency | System is unusable | %SYS-0-SHUTDOWN: System going down |
| 1 | Alert | Immediate action required | %ENV-1-FAN_FAILED: Fan failure |
| 2 | Critical | Critical conditions | %HARDWARE-2-PWR_FAIL: Power supply failure |
| 3 | Error | Error conditions | %LINEPROTO-3-UPDOWN: Interface down |
| 4 | Warning | Warning conditions | %CPU-4-HIGH_UTIL: CPU utilization high |
| 5 | Notice | Normal significant event | %LINEPROTO-5-UPDOWN: Interface up |
| 6 | Informational | General information | %SYS-6-LOGOUT: User logged out |
| 7 | Debug | Debug-level messages | %IP-7-PING: ICMP sent to 8.8.8.8 |

! Configure remote syslog Router(config)# logging host 10.0.0.201 Router(config)# logging trap informational Router(config)# logging source-interface Loopback0 Router(config)# logging on Router(config)# service timestamps log datetime msec localtime show-timezone ! Console and buffer logging Router(config)# logging buffered 65536 debugging Router(config)# logging console critical Router# show logging

## 24.3 NetFlow & IP SLA

| Technology | Purpose | Key Details |
| --- | --- | --- |
| NetFlow v5 | Traffic flow analysis and bandwidth monitoring | Exports 5-tuple flows (src/dst IP, port, protocol) to collector. UDP 2055. Read-only. |
| NetFlow v9 | Flexible NetFlow — template-based, extensible | Supports IPv6, MPLS, BGP attributes. Most widely deployed version. |
| IPFIX | Internet Protocol Flow Information Export — open standard | Based on NetFlow v9. IETF standard RFC 7011. Vendor neutral. |
| IP SLA | Active network performance testing | Sends synthetic probes to measure delay, jitter, packet loss. Used for path monitoring and failover. |
| IPSLA + Tracking | Automated failover with static routes | IP SLA tracks link quality → Object Tracking → adjusts floating static routes automatically. |

! IP SLA — Monitor WAN link health Router(config)# ip sla 1 Router(config-ip-sla)# icmp-echo 8.8.8.8 source-interface GigabitEthernet0/1 Router(config-ip-sla-echo)# frequency 10 Router(config)# ip sla schedule 1 start-time now life forever ! Track object based on IP SLA Router(config)# track 1 ip sla 1 reachability ! Floating static route that disappears when SLA fails Router(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.1 track 1 Router(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.5 5 Router# show ip sla statistics Router# show track

# Chapter 25: Multicast Routing

IP multicast delivers data from one source to a group of interested receivers efficiently. Unlike unicast (one-to-one) or broadcast (one-to-all), multicast is one-to-many with delivery only to subscribed receivers. Essential for video streaming, IPTV, and financial data feeds.

## 25.1 Multicast Addressing

| Address Range | Description |
| --- | --- |
| 224.0.0.0 – 224.0.0.255 | Link-Local Multicast — not forwarded by routers. Protocol control traffic. |
| 224.0.0.1 | All hosts on subnet |
| 224.0.0.2 | All routers on subnet |
| 224.0.0.5 | All OSPF routers |
| 224.0.0.6 | OSPF DR/BDR only |
| 224.0.0.9 | All RIPv2 routers |
| 224.0.0.10 | All EIGRP routers |
| 224.0.1.0 – 238.255.255.255 | Globally Scoped Multicast — routed on internet |
| 239.0.0.0 – 239.255.255.255 | Administratively Scoped — private multicast (like RFC 1918) |

## 25.2 IGMP — Internet Group Management Protocol

| IGMP Version | Description |
| --- | --- |
| IGMPv1 (RFC 1112) | Basic join/leave. No explicit leave — router waits for timeout. |
| IGMPv2 (RFC 2236) | Adds explicit Leave Group messages. Faster leave. Adds Group-Specific Query. |
| IGMPv3 (RFC 3376) | Source-Specific Multicast (SSM) support. Host can specify which sources to receive from. |

! Enable multicast routing Router(config)# ip multicast-routing ! Enable PIM on interfaces Router(config-if)# ip pim sparse-mode ! Configure RP (Rendezvous Point) statically Router(config)# ip pim rp-address 10.0.0.1 ! Auto-RP (Cisco) or BSR (open standard) for dynamic RP Router(config)# ip pim autorp listener ! IGMP snooping on switches (enabled by default) Switch(config)# ip igmp snooping ! Verify Router# show ip mroute Router# show ip pim neighbor Router# show ip igmp groups

# Chapter 26: SD-WAN & Network Virtualization

Software-Defined WAN (SD-WAN) decouples the WAN control plane from the data plane, enabling centralized management of multiple WAN connections (MPLS, internet, LTE) through software. It provides automated traffic routing, application-aware policies, and zero-touch deployment.

## 26.1 SD-WAN Architecture (Cisco Viptela)

| Component | Function |
| --- | --- |
| vManage | Centralized management GUI — configure, monitor, troubleshoot all devices from one dashboard |
| vSmart | Controller — distributes routing & policy information to vEdge devices. Brain of the SD-WAN. |
| vBond | Orchestrator — authenticates devices and helps them discover vSmart controllers. Initial onboarding. |
| vEdge / IOS-XE SD-WAN | Customer edge router — data plane device. Establishes IPsec tunnels to other vEdges. |
| OMP (Overlay Management Protocol) | Runs between vEdge and vSmart. Carries routes, policies, and key data. |

## 26.2 VRF — Virtual Routing and Forwarding

VRF creates multiple independent routing tables on a single router, allowing traffic isolation without separate physical devices. Used extensively in MPLS VPNs and multi-tenant environments.

! Create VRF Router(config)# vrf definition CUSTOMER_A Router(config-vrf)# rd 65001:100 Router(config-vrf)# address-family ipv4 Router(config-vrf-af)# route-target export 65001:100 Router(config-vrf-af)# route-target import 65001:100 ! Assign VRF to interface Router(config-if)# vrf forwarding CUSTOMER_A Router(config-if)# ip address 10.0.0.1 255.255.255.0 ! Ping/routing within specific VRF Router# ping vrf CUSTOMER_A 10.0.0.2 Router# show ip route vrf CUSTOMER_A

# Chapter 27: Network Automation & Programmability

Modern networks increasingly rely on automation and programmability. CCNP-level knowledge includes understanding APIs, configuration management tools, data models, and scripting to automate repetitive tasks and enable infrastructure-as-code.

## 27.1 Why Automation?

- Speed: Deploy configurations in minutes instead of hours/days

- Consistency: Eliminate human errors — same config every time

- Scalability: Manage thousands of devices with the same effort as 10

- Auditability: Version-controlled configs — know what changed and when

- Self-healing: Detect and respond to failures automatically

## 27.2 Network Programmability Stack

| Layer | Technology | Description |
| --- | --- | --- |
| Management Protocols | NETCONF, RESTCONF, gRPC, gNMI | Transport protocol for config and telemetry data |
| Data Models | YANG (RFC 7950) | Defines structure of data. Like a schema for network config. |
| APIs | REST API (JSON/XML) | HTTP-based interface to network devices |
| Encoding | JSON, XML | Data format for API payloads |
| Controllers | Cisco Catalyst Center (DNA Center), NSO, ACI | Centralized network management and automation platforms |
| Configuration Mgmt | Ansible, Terraform, Puppet, Chef, Salt | Tools for pushing configs and ensuring state |
| Scripting | Python, Go, Bash | Custom automation scripts |
| Source Control | Git, GitHub, GitLab | Version control for network configurations |

## 27.3 NETCONF & RESTCONF

| Feature | NETCONF | RESTCONF |
| --- | --- | --- |
| Standard | RFC 6241 | RFC 8040 |
| Transport | SSH (port 830) | HTTPS (port 443) |
| Data Format | XML | JSON or XML |
| Protocol | RPC-based operations | HTTP verbs (GET, POST, PUT, DELETE, PATCH) |
| Operations | get, get-config, edit-config, copy-config, lock | Standard HTTP CRUD operations on YANG paths |
| Use Case | Full configuration management; atomic transactions | RESTful access; easier for web developers |

## 27.4 Python Network Automation Example

# Using Netmiko to configure a Cisco router from netmiko import ConnectHandler device = { 'device_type': 'cisco_ios', 'host': '192.168.1.1', 'username': 'admin', 'password': 'Cisco@123', } commands = [ 'interface GigabitEthernet0/0', 'description UPLINK_TO_CORE', 'ip address 10.0.0.1 255.255.255.252', 'no shutdown', ] with ConnectHandler(**device) as net_connect: output = net_connect.send_config_set(commands) net_connect.save_config() print(output)

## 27.5 Ansible for Network Automation

# ansible-playbook configure_vlan.yml --- - name: Configure VLANs on Cisco Switch hosts: switches gather_facts: no tasks: - name: Create VLANs cisco.ios.ios_vlans: config: - vlan_id: 10 name: SALES - vlan_id: 20 name: ENGINEERING state: merged - name: Save configuration cisco.ios.ios_command: commands: write memory

🔑 KEY CONCEPT JSON vs XML: JSON is simpler and human-readable (RESTCONF default). XML is more verbose but supports namespaces and is required for NETCONF. Both carry YANG-modeled data.

PART 7: TROUBLESHOOTING & QUICK REFERENCE

# Chapter 28: Systematic Troubleshooting

A structured troubleshooting approach saves time and prevents random 'shot in the dark' fixes. The OSI model provides an excellent framework — start at Layer 1 and move up, or start at the highest layer where symptoms are visible and work down.

## 28.1 Troubleshooting Methodologies

| Method | Approach | Best Used When |
| --- | --- | --- |
| Bottom-Up | Start at Layer 1 (Physical), work up to Layer 7 | Unknown cause; physical issues suspected |
| Top-Down | Start at Layer 7 (Application), work down to Layer 1 | Application-specific problem; lower layers suspected OK |
| Divide and Conquer | Start at a middle layer (Layer 3/Network), test both directions | Experienced troubleshooter; want to quickly narrow scope |
| Follow the Path | Trace the path packet takes from source to destination | Intermittent issues; path-specific problems |
| Spot the Differences | Compare working vs. non-working configuration | Similar devices where one works and one doesn't |
| Move the Problem | Swap cables, ports, modules to isolate hardware faults | Suspected hardware failure |

## 28.2 Essential Troubleshooting Commands

| Command | Layer | What it Diagnoses |
| --- | --- | --- |
| show interfaces [name] | L1/L2 | Physical errors, duplex, speed, input/output errors, resets |
| show ip interface brief | L2/L3 | IP address, line protocol status (up/down) |
| show ip route | L3 | Routing table — is the route present? Which protocol? |
| ping [ip] | L3 | ICMP echo — basic IP reachability test |
| traceroute [ip] | L3 | Shows each hop — identify where packet is lost |
| show arp | L2/L3 | ARP cache — IP-to-MAC mappings |
| show mac address-table | L2 | Switch MAC table — where is a MAC address? |
| show spanning-tree | L2 | STP state — is a port blocked? Who is Root? |
| show vlan brief | L2 | VLAN assignments — is port in correct VLAN? |
| show interfaces trunk | L2 | Trunk ports — is VLAN allowed on trunk? |
| show ip ospf neighbor | L3 | OSPF adjacencies — are neighbors Full? |
| show ip bgp summary | L3 | BGP peers — are sessions Established? |
| show access-lists | L3–L7 | ACL hit counters — is traffic being filtered? |
| show ip nat translations | L3 | NAT table — is translation occurring? |
| debug ip packet | L3 | Real-time packet processing (use carefully!) |
| debug ip routing | L3 | Real-time routing table changes |
| show processes cpu | General | CPU utilization — identify high-CPU processes |
| show memory statistics | General | Memory utilization — identify memory leaks |
| show logging | General | Recent log messages — error history |
| show tech-support | All layers | Comprehensive output for TAC cases |

## 28.3 Common Issues & Solutions

| Problem | Symptom | Root Cause | Fix |
| --- | --- | --- | --- |
| Duplex Mismatch | High collisions, poor throughput, late collisions | One side full-duplex, other half-duplex | Set both to full-duplex or both to auto |
| VLAN Mismatch | Host can't communicate on its subnet | Port assigned to wrong VLAN | Check and correct switchport access vlan X |
| STP Loop | Broadcast storm, high CPU, all interfaces 100% | Trunk BPDU guard missing; STP disabled | Enable portfast+BPDU guard or fix topology |
| OSPF Adjacency Fail | No OSPF routes, no neighbors | Mismatched hello timers, area, auth, MTU | Compare configurations on both routers |
| Route Not in Table | Ping works to next hop but not beyond | Missing route, wrong AD, redistributed incorrectly | Check ip route, debug ip routing |
| ACL Blocking Traffic | Traffic drops, no error, hard to trace | ACL too restrictive, wrong direction (in/out) | show access-lists, remove ACL temporarily to test |
| NAT Failure | Inside hosts can't reach internet | ip nat inside/outside wrong, ACL mismatch | show ip nat trans, verify interface tags |
| Port in err-disabled | Interface goes down after few seconds | Port security, BPDU guard, or other violation | Fix root cause, then shutdown/no shutdown |

# Chapter 29: Complete Quick Reference

Well-Known Port Numbers

| Port | Proto | Service | Port | Proto | Service |
| --- | --- | --- | --- | --- | --- |
| 20 | TCP | FTP Data | 443 | TCP | HTTPS |
| 21 | TCP | FTP Control | 445 | TCP | SMB / Windows File Sharing |
| 22 | TCP | SSH | 514 | UDP | Syslog |
| 23 | TCP | Telnet | 520 | UDP | RIPv2 |
| 25 | TCP | SMTP | 546/547 | UDP | DHCPv6 Client/Server |
| 49 | TCP | TACACS+ | 636 | TCP | LDAPS |
| 53 | UDP/TCP | DNS | 646 | TCP/UDP | LDP (MPLS) |
| 67 | UDP | DHCP Server | 830 | TCP | NETCONF |
| 68 | UDP | DHCP Client | 1812 | UDP | RADIUS Auth |
| 69 | UDP | TFTP | 1813 | UDP | RADIUS Accounting |
| 80 | TCP | HTTP | 2055 | UDP | NetFlow |
| 88 | TCP/UDP | Kerberos | 3389 | TCP | RDP |
| 110 | TCP | POP3 | 4500 | UDP | IPsec NAT-T |
| 119 | TCP | NNTP | 5246 | UDP | CAPWAP Control |
| 123 | UDP | NTP | 5247 | UDP | CAPWAP Data |
| 143 | TCP | IMAP | 8080 | TCP | HTTP Alternate |
| 161 | UDP | SNMP Get/Set | 8443 | TCP | HTTPS Alternate |
| 162 | UDP | SNMP Trap | 179 | TCP | BGP |
| 389 | TCP | LDAP | 500 | UDP | IKE (IPsec Phase 1) |

IOS Privilege Levels & Access Modes

| Mode | Prompt | Privilege | Access |
| --- | --- | --- | --- |
| User EXEC | Router> | Level 1 | Basic commands: ping, show version, telnet |
| Privileged EXEC | Router# | Level 15 | All show commands, debug, configure terminal |
| Global Config | Router(config)# | Level 15 | System-wide configuration changes |
| Interface Config | Router(config-if)# | Level 15 | Per-interface settings |
| Router Config | Router(config-router)# | Level 15 | Routing protocol configuration |
| Line Config | Router(config-line)# | Level 15 | Console, VTY, AUX line settings |
| VLAN Config | Switch(config-vlan)# | Level 15 | VLAN creation and naming |

Routing Table Code Reference

| Code | Meaning | Code | Meaning |
| --- | --- | --- | --- |
| C | Directly Connected | O | OSPF |
| L | Local (specific interface IP /32) | O E1 | OSPF External Type 1 |
| S | Static | O E2 | OSPF External Type 2 |
| S* | Static default route (gateway of last resort) | O IA | OSPF Inter-Area |
| D | EIGRP | B | BGP |
| D EX | EIGRP External (redistributed) | i | IS-IS |
| R | RIP | EX | External (redistributed into IS-IS) |
| + | Candidate default | * | Best path among equal-cost routes |

IPv4 Subnetting Complete Reference

| Prefix | Mask | Wildcard | Hosts | Block |
| --- | --- | --- | --- | --- |
| /8 | 255.0.0.0 | 0.255.255.255 | 16,777,214 | 16,777,216 |
| /16 | 255.255.0.0 | 0.0.255.255 | 65,534 | 65,536 |
| /17 | 255.255.128.0 | 0.0.127.255 | 32,766 | 32,768 |
| /18 | 255.255.192.0 | 0.0.63.255 | 16,382 | 16,384 |
| /19 | 255.255.224.0 | 0.0.31.255 | 8,190 | 8,192 |
| /20 | 255.255.240.0 | 0.0.15.255 | 4,094 | 4,096 |
| /21 | 255.255.248.0 | 0.0.7.255 | 2,046 | 2,048 |
| /22 | 255.255.252.0 | 0.0.3.255 | 1,022 | 1,024 |
| /23 | 255.255.254.0 | 0.0.1.255 | 510 | 512 |
| /24 | 255.255.255.0 | 0.0.0.255 | 254 | 256 |
| /25 | 255.255.255.128 | 0.0.0.127 | 126 | 128 |
| /26 | 255.255.255.192 | 0.0.0.63 | 62 | 64 |
| /27 | 255.255.255.224 | 0.0.0.31 | 30 | 32 |
| /28 | 255.255.255.240 | 0.0.0.15 | 14 | 16 |
| /29 | 255.255.255.248 | 0.0.0.7 | 6 | 8 |
| /30 | 255.255.255.252 | 0.0.0.3 | 2 | 4 |
| /31 | 255.255.255.254 | 0.0.0.1 | 2 (PPP) | 2 |
| /32 | 255.255.255.255 | 0.0.0.0 | 1 (host) | 1 |

OSI Layer Attacks & Defenses

| OSI Layer | Attack Type | Description | Defense |
| --- | --- | --- | --- |
| L1 Physical | Cable cutting / jamming | Physical destruction or RF jamming | Physical security; redundant paths |
| L2 Data Link | MAC Flooding | Overflow CAM table → switch acts as hub | Port Security |
| L2 Data Link | VLAN Hopping | Access unauthorized VLANs via double-tagging or DTP | Disable DTP; change native VLAN; use unused VLAN |
| L2 Data Link | Rogue DHCP | Fake DHCP server assigns wrong gateway (MiTM) | DHCP Snooping |
| L2 Data Link | ARP Spoofing | Fake ARP replies → traffic redirected to attacker | Dynamic ARP Inspection (DAI) |
| L2 Data Link | STP Attack | Become Root Bridge to control traffic path | Root Guard; BPDU Guard; BPDU Filter |
| L3 Network | IP Spoofing | Forge source IP to bypass ACLs | IP Source Guard; uRPF |
| L3 Network | Route Injection | Inject false routes into routing protocols | OSPF/BGP authentication; prefix filtering |
| L3 Network | Smurf Attack | Broadcast ping with spoofed source → amplification | Disable directed broadcasts: no ip directed-broadcast |
| L4 Transport | SYN Flood (DoS) | Half-open TCP connections exhaust server resources | SYN cookies; rate limiting; firewall |
| L7 Application | DNS Poisoning | Inject false DNS records → redirect to fake site | DNSSEC; use trusted resolvers |

CCNA 200-301 vs CCNP ENCOR 350-401 Topics

| Topic Area | CCNA 200-301 | CCNP ENCOR 350-401 |
| --- | --- | --- |
| Routing | Static, OSPFv2, basics of EIGRP/BGP | OSPF multi-area, EIGRP, BGP attributes & path selection, route redistribution, PBR |
| Switching | VLANs, STP, EtherChannel basics | MSTP, advanced STP, L3 switching, VRF |
| WAN & VPN | WAN types overview, GRE basics | DMVPN, FlexVPN, IPsec deep dive, SD-WAN architecture |
| Wireless | 802.11 standards, WPA2/WPA3, basic WLC | RF design, WLC redundancy, 802.1X, advanced WLAN |
| Security | ACLs, SSH, port security, basic AAA | AAA deep dive, TrustSec, 802.1X with ISE, CoPP |
| QoS | QoS concepts, marking basics | Full MQC, CBWFQ, LLQ, DSCP, queuing deep dive |
| Architecture | SDN overview, automation intro | Cisco DNA, LISP, VXLAN, ACI concepts |
| Automation | JSON/REST basics, Python overview | YANG, NETCONF, RESTCONF, Ansible, Python scripting |
| Protocols | IPv4/6, DNS, DHCP, NTP, SNMP, Syslog | MPLS, multicast PIM, advanced BGP, IS-IS intro |

Study Roadmap: Beginner to CCNP

| Phase | Duration | Focus | Resources |
| --- | --- | --- | --- |
| Phase 1 — Foundations | 2–4 weeks | OSI, TCP/IP, IPv4, subnetting, devices, cables | Chapters 1–4 of these notes; Boson Exam Sims |
| Phase 2 — Switching | 3–4 weeks | VLANs, STP, EtherChannel, L2 security, RSTP | Chapters 5–9; Cisco Packet Tracer labs |
| Phase 3 — Routing | 4–6 weeks | Static, OSPF, EIGRP, ACLs, NAT, IPv6 routing | Chapters 10–17; GNS3 or CML labs |
| Phase 4 — Services | 2–3 weeks | DHCP, DNS, NTP, SNMP, Syslog, QoS, Security | Chapters 18–20; Cisco DevNet Sandbox |
| Phase 5 — WAN & Wireless | 2–3 weeks | WAN types, VPN, IPsec, 802.11, WLC, CAPWAP | Chapters 21–23; virtual lab environments |
| Phase 6 — CCNA Practice | 3–4 weeks | Full practice exams; weak area review; flash cards | Cisco Official Study Guide; Boson; 9tut.com |
| Phase 7 — CCNP Prep | 3–4 months | BGP, OSPF multi-area, redistribution, automation | Chapters 24–27; Cisco Press ENCOR study guide |

You Now Have Everything You Need! CCNA 200-301 · CCNP ENCOR 350-401 Study Daily • Build Labs • Understand Concepts • Succeed! 🎯